UNIVERSITÀ DEGLI STUDI DI PADOVA

DIPARTIMENTO DI INGEGNERIA INDUSTRIALE

LAUREA MAGISTRALE IN INGEGNERIA CHIMICA E DEI PROCESSI INDUSTRIALI

Tesi di Laurea Magistrale in

Ingegneria Chimica e dei Processi Industriali

DESIGN AND VALIDATION OF A MICROFLUIDICS BASED BLOOD BRAIN BARRIER (BBB)-ON-A-CHIP FOR IN VITRO GLIOBLASTOMA CANCER TREATMENT

Relatore: Prof.ssa Elisa Cimetta

Correlatore: Ing. Sara Micheli

Laureando: MARTINA DE MARCHI

ANNO ACCADEMICO 2023-2024

# Abstract

The blood-brain barrier (BBB) is a dynamic interface between capillary blood flux and the central nervous system (CNS), allowing the passage of essential nutrients for the normal metabolism of brain cells and providing protection against many toxic compounds and pathogens. The BBB is primarily formed by endothelial cells, Astrocytes and Pericytes. The above-mentioned protection against toxic compounds however also limits the penetration of chemotherapy drugs into the brain and thus reduces the efficacy of chemotherapy for brain tumors. For the development of better drugs and improved therapeutic approaches, it is thus of paramount importance to study the interactions between selected compound and the BBB in vitro. The application of microfluidic technology provides a promising avenue for replicating the BBB. The objective of this study is to design and create a microfluidic BBB device for investigating treatment strategies for glioblastoma cancer.

The microfluidic BBB-on-a-Chip developed in this work is composed by two PDMS layers separated by a PETE membrane with 3 μm pores. The layers are hydraulically sealed through plasma treatment. The top horizontal microchannel has an inlet and an outlet and has the membrane at its bottom surface, the other side of the membrane is in direct contact with the bottom channels. The lower layer has three microwells, fluidically connected to the lower channel, representing the tumor area (Glioblastoma spheroids). Cells are seeded on both sides of the membrane: HUVECs on the top and a coculture of Astrocytes and Pericytes on the bottom. The Live & Dead assay indicates cell viability up to one week. Staining with Phalloidin and DAPI highlights the presence of actin filaments, a protein that constitutes the cell's cytoskeleton. Immunofluorescence analysis with CD31 and GFAP monoclonal antibodies reveals the formation of the endothelial barrier and the presence of Astrocytes, respectively. The BBB-on-chip model has been also validated in term of permeability and Trans Endothelial Electrical Resistance (TEER). The measured permeability is comparable to the values reported in the literature and resembles the barrier present in the human body. The TEER values, increasing over the course of 7 days in culture, also confirmed the formation of the barrier. In the final stage of the study, 3D Glioblastoma spheroids are formed and characterized to validate the 3D tumor model. Moreover, spheroids are seeded in the dedicated area of the BBB microfluidic device. Subsequently, different drugs are fluxed into the upper part of the device to test their capability to cross the membrane and reach the cancer spheroids area.

# Riassunto

La barriera emato-encefalica (BEE) è un’interfaccia dinamica tra il flusso sanguigno capillare ed il sistema nervoso centrale (SNC) che permette il passaggio di nutrienti essenziali per il normale metabolismo delle cellule cerebrali e fornisce protezione da molteplici composti tossici e patogeni. La BEE è principalmente formata da cellule endoteliali, Astrociti e Periciti. Questa protezione contro i composti tossici, tuttavia, limita anche la penetrazione dei farmaci chemioterapici nel cervello e quindi riduce l'efficacia della chemioterapia per i tumori cerebrali. Per lo sviluppo di migliori farmaci e approcci terapeutici, è quindi di fondamentale importanza studiare le interazioni tra il composto selezionato e la BBB in vitro. L’utilizzo della tecnologia microfluidica permette di riprodurre la barriera emato-encefalica (BEE) in vitro. L’obiettivo di questo studio è il design e la realizzazione di un dispositivo microfluidico (BBB-on-chip) atto a ricreare un modello di BEE per lo studio di nuove strategie nel trattamento del glioblastoma. La BEE microfluidica realizzata in questo lavoro è composta da due strati di PDMS separati da una membrana porosa in PETE, i cui pori hanno un diametro di 3 μm. Questi strati sono sigillati tra loro attraverso un trattamento al plasma. Lo strato superiore del dispositivo (comparto del flusso sanguigno capillare) è costituito da un canale dotato di un ingresso e di un’uscita ed è chiuso inferiormente dalla membrana che è a sua volta a diretto contatto con lo strato inferiore (comparto del sistema nervoso) anche esso dotato di un ingresso e uscita indipendenti. Nello strato inferiore sono presenti tre micro-pozzetti ciascuno collegato con il canale inferiore che rappresentano la zona tumorale dove verranno posizionati gli sferoidi di Glioblastoma.

Nel corso del progetto è stato ideato e ottimizzato il protocollo di semina delle 3 linee cellulari costituenti la BEE all’interno del dispositivo microfluidico per ottenere la formazione della barriera: la linea di cellule endoteliali (HUVEC) viene seminata al di sopra della membrana utilizzando l’ingresso del canale, mentre una bicoltura di astrociti e periciti viene seminata utilizzando l’ingresso dello strato inferiore del dispositivo capovolgendo il dispositivo per promuovere la loro adesione allo strato inferiore della membrana. La vitalità delle tre linee cellulari è stata analizzata tramite Live&Dead, mostrando come il dispositivo costituisca un ambiente favorevole alla coltura cellulare. Inoltre, tramite immunofluorescenza sono stati marcati gli anticorpi CD31 per le HUVEC e GFAP per gli Astrociti, confermando la formazione della membrana dopo 7 giorni di coltura nel dispositivo. Il modello microfluidico di BEE è stato anche validato in termini di permeabilità e di resistenza elettrica transepiteliale (TEER). La permeabilità ottenuta è comparabile con i valori riportati in letteratura e sono simili a quelli riscontrati in vivo. I valori di resistenza elettrica transepiteliale ottenuti aumentano nel corso dei 7 giorni di coltura, confermando la formazione della barriera. Per completare lo studio, dopo aver ottimizzato il protocollo di formazione degli sferoidi tridimensionali di Glioblastoma, questi vengono caratterizzati al fine di validare il modello 3D di tumore. Gli sferoidi sono stati inoltre integrati nel dispositivo utilizzando i pozzetti e sono stati lasciati in perfusione per 24 ore confermando la loro vitalità nel sistema che risulta quindi uno strumento utile allo studio dell’interazione di farmaci attraverso la BEE per il trattamento del Glioblastoma.

Table of Contents

Introduction	3

Chapter 1	3

State of the art	3

1.1 Microfluidics	3

1.1.1 Transport Phenomena in microfluidics	3

1.1.1.1 Navier-Stokes equation	3

1.1.1.2 Reynolds number	4

1.1.1.3 Laminar flow	5

1.1.2 Microfluidics in biology	6

1.2 Microfluidics: Manufacturing techniques and materials	7

1.2.1 Photolithography	7

1.2.2 Replica molding	9

1.2.3 PDMS	9

1.2.4 Plasma Treatment	10

1.3 Blood brain barrier	11

1.4 Microfluidic model for BBB	13

1.4.1 Design principles for BBB microfluidic models	13

1.4.2 Cell type used to model BBB on chip	15

1.4.2.1 Non-cerebral cells origin	15

1.4.2.3 Cerebral cells	15

1.4.2.4 Immortalized cell lines	16

1.5 Glioblastoma cancer	16

1.6 3D Tumor Spheroid to cancer disease modelling	17

1.7 Aim of the thesis	20

Chapter 2	21

Materials and methods	21

2.1 Design and production of the microfluidic platform	21

2.1.1 Design principle of the BBB-on-chip	21

2.1.2 Photolithography	22

2.1.2.1 Silicon wafer pretreatment and photoresist deposition	23

2.1.2.2 Soft Bake and UV exposure	23

2.1.2.3 Post Exposure Bake, Development and Hard Bake	25

2.1.3 PDMS Replica molding	25

2.1.4 Plasma bonding	26

2.2 Leakage test of the BBB-on-chip	27

2.3 Cell Cultures	27

2.3.1 Thawing and maintenance of cells	27

2.3.1.1 Astrocytes	28

2.3.1.2 Pericytes	29

2.3.1.3 HUVEC	29

2.3.2 Cell count	30

2.4 BBB-on-chip microfluidic model: cells seeding	32

2.5 GelMA 3D model to mimic the BBB	34

2.6 Biological validation of the BBB model	36

2.6.1 Live & Dead assay	36

2.6.2 Fixation protocol using PFA	37

2.6.2.1 Phalloidin & DAPI	37

2.6.2.2 Immunofluorescence	37

2.7 Determination of paracellular barrier characteristics in vitro	38

2.7.1 Measurement of Permeability	39

2.7.2 Measurement of TEER	40

2.8 3D Glioblastoma spheroids experiments	42

Chapter 3	45

Results and discussion	45

3.1 2D BBB-on-chip microfluidic device validation	45

3.2 Biological Validation for BBB-on-chip results	46

3.2.1 Astrocytes and Pericytes seeding inside the BBB-on-chip	46

3.2.2 HUVECs seeding inside the BBB-on-chip	48

3.2.3 Co-culture of Astrocytes and Pericytes inside the BBB-on-chip	49

3.2.4 Triculture model of the BBB-on-chip	50

3.3 Results for permeability test	55

3.4 Results for TEER experiments	57

3.5 3D GelMA BBB validation results	58

3.6 Glioblastoma spheroids results	59

3.6.1 GB spheroid characterization	59

3.6.2 Spheroids experiment in the BBB-on-chip	61

Conclusions	63

References	65

# Introduction

Microfluidics emerged in the beginning of the 1950s, and is a branch of fluid dynamics which studies the manipulation of small volumes of fluids using devices with channels sized from ten to hundreds of micrometers. The behavior of fluids at the microscale differs from the macroscale opening possibilities to develop new solutions.

Microscale systems are characterized by small transport distances, small handling volumes, easily predicted and mathematically described transport phenomena, which translate into precise control of culture conditions and reduce the intrinsic variability of biological phenomena. The combination of Lab-on-Chip and Biology leads to the field of Organ-on-chip (OOC), which involves a microfluidic device where specific cells are cultured, mimicking activities and physiological responses of the native human organ. Thanks to this technology, animal experimentation can be reduced, and moreover, due to the small volumes of reagents needed for a micro-bioreactor, costs are also greatly reduced, leading to high-throughput screening platforms.

This innovative project was developed in the BIAMET laboratory (Biomedical Applications of Multiscale Engineering Technologies), where engineering principles are applied to in vitro biological studies to test therapeutic anticancer treatments.

Glioblastoma is the most malignant brain tumor which primarily affects adults. It is the most common among glial neoplasms and it usually occurs in the cerebral hemispheres. In most cases, Glioblastoma appears suddenly, and generally, the mean survival time is between 6 and 12 months.

The blood brain barrier (BBB), already hypothesized in the second half of the 19th century, insulates the brain environment from the blood, and it is the main cause of the drug-resistance of glioblastoma, as it prevents the delivery of chemotherapy agent to the tumor site. The aim of this work is to design and realize a BBB-on-a-chip for investigate new treatment strategies which involve loaded monocytes that genetically influence Glioblastoma cells to make them more receptive to standard treatments. The thesis is divided into three Chapters where in the First Chapter the State of the Art of Microfluidics and of the different way of BBB realization are enounced; in the Second Chapter all the materials and methods used are described and in the Third Chapter results about all tests to validate the BBB-on-a-chip are discussed.

# Chapter 1

# State of the art

In this First Chapter, a general overview of microfluidics and its role in the biomedical research field will be presented. Attention will be focused on specific microfluidic devices, their materials and technologies, production techniques, and the advantages associated with their use. Subsequently, the chapter will include a description of the Blood-Brain Barrier (BBB) and the various existing microfluidic devices designed to mimic it. Finally, the motivation and aims of the thesis will be explained.

# 1.1 Microfluidics

Since the 1990s, attempts have been made to implement a process of miniaturization in the field of biological and chemical analysis. Biological microfluidics, an emerging multidisciplinary science, intersects the fields of biotechnology, biochemistry, chemistry, nanotechnology, and physics to create devices that control the microenvironment of cell culture1. For the application at the core of this thesis project, the most relevant feature is the observation that the physical phenomena typically characterizing the macroscale dramatically change at the microscale. In the macroscale, for example, mass transport is usually dominated by inertial forces, while viscous dissipation is negligible; in microfluidic devices, the situation is just the opposite. At the microscale, physical phenomena that are usually insignificant at the macroscale become predominant, offering various advantages2.

## 1.1.1 Transport Phenomena in microfluidics

In this paragraph an overview of all the governing equations of microfluidics will be shown and explained, considering the typical simplifications which well represent fluids into a microscopic environment.

### 1.1.1.1 Navier-Stokes equation

Fluids are considered continuous materials, so the velocity field of a Newtonian fluid obeys to Navier-Stokes Equation (1.1)1:

|  | ρ ∂u∂t+u·∇u =∇·σ+f=-∇p+∇2u+f . | (1.1) |
| --- | --- | --- |

In the compact version of Navier-Stokes equation, the inertial acceleration term stays on the left and forces appear on the right1. In microfluidics, the inertial forces are significantly smaller than viscous forces and so the non linear term can be neglected, as shown in Equation (1.2)1:

|  | ρ ∂u∂t =∇·σ+f=-∇p+∇2u+f . | (1.2) |
| --- | --- | --- |

The law of conservation of mass, represented in the Equation (1.3), states that1:

|  | ∂ρ∂t+∇·ρu=0. | (1.3) |
| --- | --- | --- |

The condition of incompressibility, for which density can be assumed constant, is represented by the Equation (1.4)1:

|  | ∇·u=0. | (1.4) |
| --- | --- | --- |

In microfluidic devices transport phenomena such as diffusion or convection can take place, whose relative importance is valued through a ratio expressed by adimensional numbers1. The most important adimensional numbers are1:

- Reynolds number which expresses the ratio between inertial forces and viscous forces;

- Péclet number which quantifies the relationship between convection and diffusion;

- Grashof and Rayleigh numbers that consider buoyancy forces;

- Knudsen number which relates macroscopic and microscopic scales.

### 1.1.1.2 Reynolds number

The Reynolds number (Re), shown in the Equation (1.5) is the most mentioned in microfluidics and it depends on the specific properties of the fluid like density, viscosity, mean velocity and diameter of the microchannel1:

|  | fifv=ρU₀L₀=Re . | (1.5) |
| --- | --- | --- |

Particularly, Reynolds number describes the regime of the fluid analyzed which can be laminar, transient or turbulent1:

- if Re < 2100 the flow is laminar;

- if 2100 < Re < 4000 the flow is transient;

- if Re > 4000 the flow is turbulent.

Almost all microfluidic platforms use fluids with Re small enough to make inertial effects irrelevant1. For common microfluidic devices, which employ water as a typical working fluid, mean velocities range from 1 μm/s up to a 1 cm/s, with a channel radius from 1 to 100 μm, so Reynolds numbers are between 10-6 and 101. These low Reynolds values imply that viscous forces normally dominate over inertial forces and that the resulting flows are linear1. In fact, when Re is very small, the non-linear terms disappear resulting in a linear and predictable Stokes flow, however as Re increases, some characteristics of inertia become evident1.

Since most of the time microchannels have a rectangular section, Re is expressed with another notation, expressed into the Equation (1.6)2:

|  | Re ≡ ρvDeq μ , | (1.6) |
| --- | --- | --- |

where ρ is the fluid density, v is the characteristic velocity of the fluid, μ is the fluid viscosity and Deq is the equivalent diameter, calculated through the Equation (1.7):2

|  | Deq= 4SP , | (1.7) |
| --- | --- | --- |

where S is the section of the microchannel, P is the wet perimeter2.

### 1.1.1.3 Laminar flow

Considering constant density and viscosity, Reynolds has experimentally demonstrated that a fluid in a cylindrical duct, in a range of low average speeds, thus in a laminar regime, maintain its intact and straight layers over time3. The velocity profile is parabolic and the flow rate is described by the Haegen-Poiseuille law3. This law allows to relate the pressure drop and the flow rate inside a duct of an incompressible fluid in laminar flow, as in the Equation (1.8)3:

|  | ∆p= 8LQπ4R , | (1.8) |
| --- | --- | --- |

where ∆p is the pressure drop,  is the fluid viscosity, L is the length at which the pressure drop is determined, R is the radius of the duct and  Q is the volumetric flow rate per unit of time3.

On the other hand, turbulent flow is a regime in which the fluid moves in a random and unpredictable way, creating vortices3. It is impossible to describe the position of a fluid particle as a function of time, and the velocity profile in the duct is no longer parabolic, as its shape becomes the so-called plug flow3. In microfluidics, the difficulty of describing the turbulent regime with a mathematical model, does not exist due to the small dimensions of channels whose order of magnitude is μm, in fact the flow is always laminar3.

Diffusion phenomena is originated from the gradient of a specific property along one direction and it is associated with the Brownian flow of particles which diffuse in order to fill the gap2.

If the concentration gradient of the species A is considered, on stationary conditions, Fick’s law, shown in the Equation (1.9) is valid2:

|  | JA=DAB - ∂CA∂y . | (1.9) |
| --- | --- | --- |

The diffusion along one direction can be represented also with the Equation (1.10)2:

|  | d2=2Dt . | (1.10) |
| --- | --- | --- |

Where d is the distance covered by a particle over time t and D is the diffusion coefficient2.

So, since the time increases with the square of the distance, diffusion becomes very important on the microscale as it takes a few seconds for distances of the order of a micron2.

For example, hemoglobin, which has a diffusion coefficient D equal to 7 ∙ 10-7 cm2/s in water, takes 106 s to diffuse 1 cm, but only 1 s to diffuse along 1 μm2.

Diffusion times are therefore very short in microdevices and for this reason microchannels can be used to create concentration gradients with particular profiles2.

## 1.1.2 Microfluidics in biology

Over the last 10 years, academic and industrial interest in microfluidic applications has grown because conventional in vitro techniques are not able to reproduce the entire in vivo environment, characterized by different cell types, extracellular matrix and other interactions and signals4. The aim of biology is to study inter and intracellular interactions and better understand the complicated network of regulating functions4. Therefore, microfluidic devices are an interesting method to solve these issues, thanks to numerous advantages listed herein5:

- Microfluidic devices are compact systems, resulting in smaller volumes of reagents used, less waste and lower costs compared to the use of traditional cell culture methods as Petri dishes, flasks and multiwells;

- The laminar flux within the microchannels leads to establish precise concentration gradients and/or a specific cells distribution;

- Microdevices are high-throughputs screening (HTS) platforms, which means that the yield increases, because the system feedback is faster, so the control of the entire process is higher;

- The use of transparent biocompatible materials to produce these platforms allow to control the experiments in real time through microscope;

- Fabrication costs are low and devices are disposable;

- The microenvironment can be easily controlled through the possibility to integrate valves, pumps and mixer directly on chip.

However, there are some drawbacks. For example, considering the fabrication process, imperfect adherence between the platform’s layers can make it impossible to use the device. Another issue is the formation of bubbles inside the microchannels that can prejudice the success of the experiment. Additionally, the faster consumption of nutrients by cells and the increase of metabolites, due to the small volume of the chamber, can lead to apoptosis if medium is not supplied as soon as possible5.

Despite these drawbacks, microfluidics allows for the creation of the so-called “lab on a chip”, integrating multiple aspects of modern biology or chemistry laboratories into a single microdevice5.

# 1.2 Microfluidics: Manufacturing techniques and materials

Regarding production methods, the techniques mainly used include micromachining, soft lithography, embossing, in situ construction, injection molding, and laser ablation2. Depending on the specific application, each of the listed techniques could be the most suitable for device fabrication2. The original materials used in microfluidics were silicon and glass; however, they have been largely replaced by plastics. Neither glass nor silicon possess all the properties required for working with living cells: silicon is expensive and opaque, rendering it unsuitable for use with traditional detection methods. Additionally, it is easier to produce components using elastomers than rigid materials. For biological purposes, the most commonly used material is a silicon-based organic polymer called Polydimethylsiloxane (PDMS), which is optically transparent, soft, and characterized by low cost and biochemical compatibility6.

## 1.2.1 Photolithography

Photolithography is a technique to model microchannels and microwells on a thin film of Photoresist deposited on a silicon support, called wafer6. Microstructure’s design is obtained using the software AutoCAD and then printed on the photomask6. The photomask is a plate with transparent areas that allow light to pass throw the specific pattern, while the photoresist is a light-sensitive material6. UV rays impress the desired geometry from the photomask to the Photoresist6. On the market there are two types of photoresist, which are represented in Figure 1.17:

- positive: resist does not polymerize when exposed to light, remaining soluble with the developer;

Figure 1.1 Differences between positive and negative photoresist when exposed to the developer. Adapted from7.

negative: the portion of resist which is not exposed to UV light, does not polymerize and can be removed.

The procedure involves pouring the photoresist onto the wafer and making a completely uniform thin layer through the rotating movements of the spin coater7. Here, the time and temperature combination depend on the desired final thickness of the layer of photoresist that we want to obtain over the master7. Subsequently the photomask is placed over the resist layer, and it is exposed to UV light with specific time and power setting7. Then, the mask is raised, and the unpolymerized resin is selectively removed by the developer while the polymerized geometry remains in relief7.

Figure 1.2 Photolithography and Replica molding processes. Adapted from8.

The master is obtained, and it can be used to produce the microfluidic device through replica molding technique as schematized in Figure 1.28.

## 1.2.2 Replica molding

Replica molding is a particular technique used in microscale manufacturing. It involves creating one or more replicas of a desired pattern impressed into the master mold (Figure 1.3). Once the master is obtained through photolithography, PDMS is poured onto the master and it is cross-linked at about 65 °C for 1 hour6. Then, the PDMS layer is easily removed from the master, obtaining the desired geometry6. Usually, a microfluidic device is made of a PDMS layer that can be attached to a support, such as a microscope slide, creating the internal rectangular chambers.

The watertight closure is achieved through plasma treatment, which creates a covalent bond between the two PDMS layers.

## 1.2.3 PDMS

Polydimethylsiloxane (PDMS) is a silicone polymer which is highly suitable for the construction of microfluidic devices6. It possesses multiple properties such as flexibility and optical transparency, allowing for the use of various optical observation methods6. Additionally, PDMS is biocompatible, being non-toxic to cells, permeable to gases and impermeable to water6. Moreover, PDMS is characterized by remarkable temperature, chemical and oxidation resistance, excellent electrical insulation and resistance to aging6. Its greatest advantages lie in its affordability and ease of bonding to other surfaces6. To obtain PDMS mixture, a curing agent and a base are vigorously mixed in a 1:10 ratio6. Then, this mixture is poured onto the mold and cured at 65-70 °C in the oven6. The curing time

varies depending on the desired final mechanical properties6. At the end of the curing process, the PDMS layer is lifted from the mold, resulting in a replica of the geometry present on it6.

Generally, PDMS devices can be bonded either reversibly, using Van der Waals forces (conformal contact) or irreversibly, undergoing plasma treatment which forms covalent bonds4. PDMS is formed by the repetition of the monomer (C2H6OSi)n where CH3 groups make its surface hydrophobic, leading to poor wettability with aqueous solvents4. Therefore, PDMS can physically absorb hydrophobic solvents such as toluene, causing the polymer to swell4. However, by using different chemical modifications such as plasma activation, ultra violet (UV) irradiation, piranha etching, silanization, graft polymerization, chemical vapor deposition (CVD), sol–gel chemistry or 3D scaffold formation (e.g. hydrogels), the wettability can be effectively altered and the surface chemistry of the PDMS microchannels can be tailored, to fulfill specific biotechnological requirements4. Additionally, functionalization of the microchannels with different functional groups such as hydroxyl and thiol groups, epoxy rings, NHS-esters, amine groups, carboxylic and acrylic groups, aldehyde groups, carbodiimide chemistry, maleimide groups, or isocyanate and isothiocyanate groups result in covalent immobilization of biological entities on the channels4.

From a biological standpoint, the hydrophobic nature of PDMS can be an issue in biotechnological applications as it causes non-specific adsorption of biomolecules, reducing the device's efficacy when it is implemented for cell culture applications and bioassays4. Although the hydrophobicity of PDMS surfaces is not favorable for cell adhesion, it can be exploited for the physical adsorption of extra cellular matrix (ECM) proteins, which effectively promote cell adhesion and growth4. For instance, a coating of PDMS microchannels with fibronectin diluted in PBS solution is often recommended to

Figure 1.3 Irreversible covalent bonds between PDMS and glass. Adapted from10.

induce biocompatibility for some specific cell culture4.

## 1.2.4 Plasma Treatment

Plasma cleaning is an effective, economical and environmentally safe method for surface preparation which eliminates oils and grease at the nano-scale and reduces contamination up to 6 times with respect to traditional solvent cleaning9. Oxygen plasma produces a pristine surface, ready for bonding or further processing, without any harmful waste material9. Ultra-violet light generated in the plasma chamber breaks organic bonds of surface contaminants and energetic oxygen species created in the plasma react with organic contaminants to form mainly water and carbon dioxide9. These byproducts are continuously removed and pumped away from the chamber9. Plasma treatment is also able to activate a specific surface (of a wide range of materials) to improve its adhesion characteristics, making them easier to bond. Many polymers such polyolefins, like polyethylene and polypropylene, are chemically inert due to the absence of polar and reactive functional groups in their structure9. For this reason, they cannot be bonded easily to other materials, exhibiting poor adhesion even with glues9. Conversely, plasma surface activation alters the surface of the polymer, by attaching polar or functional groups to it, making many polymers receptive to bonding agents and coatings. Oxygen is usually used as the process gas; however, many plasma activations can also be carried out with just ambient air9. Afterwards, the components treated with plasma remain active for a few minutes up to several months, depending on the particular material considered. In microfluidics, plasma treatment is used to create strong covalent bonds between a PDMS platform and a glass slide (as reported in Figure 1.3), or another PDMS element, improving adhesion properties and so making a perfect watertight closure10. As previously mentioned PDMS is made of repeated units of —O—Si(CH3)2—, which exposed to oxygen plasma develop silanol groups (—OH) at the expense of methyl groups —(CH3) 11. So, the oxidation of the surface layer increases the concentration of hydroxyl groups and this leads to the formation of strong intermolecular bonds11. When the two layers are brought into conformal contact, these silanol groups bond with those of the other activated PDMS surface11. For both PDMS and glass, these reactions yield Si-O-Si bonds after the loss of a water molecule11. These covalent bonds form the basis of a tight irreversible seal between the layers11.

# 1.3 Blood brain barrier

The BBB belongs to the neurovascular unit (NVU) and consists of Astrocytes, Pericytes, glial cells, neuronal cells and endothelial cells (ECs) that form capillary and extracellular matrix (ECM) proteins as reported in Figure 1.412. The brain capillaries are composed of ECs tightly linked together and surrounded by Pericytes and a basement membrane (with a thickness of 30 to 40 nm) which includes collagen IV, laminin glycoproteins, proteoglycans, and fibronectin13.

The blood-brain barrier (BBB) is an active, dynamic and complex interface between the blood circulation and the central nervous system (CNS). It transports nutrients essential for the normal metabolism of brain cells, regulates the passage of ions from the blood into the brain and provides protection against many toxic compounds and pathogens13. The BBB restricts the free movement of different substances between the two compartments and plays a crucial role in the maintenance of the homeostasis of the CNS14. According to recent research, 98% of small-molecule and 100% of large molecule drugs cannot cross the BBB 13.

Figure 1.4 Schematics of BBB and its components: astrocytes, pericytes and endothelial cells. Adapted from12.

The BBB has a barrier and a carrier function14. The barrier function means that the BBB restricts the transport of potentially toxic or harmful substances from the blood to the brain. This action is achieved through four defense lines14:

- The paracellular barrier, formed by interendothelial junctions, restricts the free movement of water-soluble compounds between two adjacent cells14;

- Endocytosis is a process through which the cell internalizes molecules present in the extracellular space, creating a vesicle to enclose it. Transcytosis, on the other hand, is a cellular process through which different macromolecules are transported from one side of the cell to another through the cytoplasm of the cell itself. The transcellular barrier is made possible by the low level of endocytosis and transcytosis, typical of brain endothelial cells14;

- The enzymatic barrier is provided by a complex set of drug metabolizing enzymes, capable to degrading different chemical compounds14;

- In addition to these, the cerebral endothelium expresses a large number of efflux transporters. Active efflux transport systems in the blood-brain barrier work as a detoxification system in the brain14.

The carrier function regulates the transport of nutrients to the brain and the removal of metabolites. Small lipid-soluble molecules and blood gases (such as oxygen and carbon dioxide) diffuse passively into the BBB, while essential polar nutrients like glucose and amino acids require specific transport proteins to reach the brain14. Since the BBB is responsible of many phenomena, there is a need to develop new tools that are scalable, cost effective and clinically relevant to study the effect of specific drugs into the CNS. Moreover, major brain disorders as Traumatic brain injury (TBI), brain tumors and Alzheimer's disease (AD) would beneficiate from in vitro BBB/NVU models.13

Generally, the BBB itself is one of the major impediments in the treatment of CNS disorders due to its relative impermeability which prevents many drugs from reaching therapeutic concentrations in the CNS, but it is important to note that in presence of a large number of neurological disorders like cerebral ischemia, brain trauma, tumors and other neurodegenerative disorders, the permeability of the BBB increases14. Nowadays, also brain cancer represents one of the leading causes of mortality13. Despite surgical treatment, chemotherapy and radiotherapy, patients survive only for a short time13. A novel strategy to treat various tumors, also in relapsed patients, is the biological therapy which uses the body’s immune system to kill cancer cells, and to prevent or slow the spread of cancer13. However, an inefficient delivery of therapeutic agents to the tumor sites due to the BBB represents a major challenge in cancer biological therapy13.

# 1.4 Microfluidic model for BBB

In this paragraph, the different existing devices capable of replicating the BBB will be discussed. It will cover the cell types used to obtain the BBB in vitro, as well as the protocols used in the literature to validate it.

## 1.4.1 Design principles for BBB microfluidic models

In many works, the dynamic microfluidic BBB-on-a-chip contains at least two compartments: a vascular compartment and a brain compartment, where specific cell cultures are seeded15. The flow rate in the compartmentalized microfluidic BBB has to simulate the physiological blood flow in the vascular lumen to maintain the proper shear stress, which is necessary to form a tightly connected functional endothelial barrier15. A microfluidic BBB-on-a-chip should simulate the following characteristics in vitro:

- 3D vascular-like structure similar to that in vivo,

- interactions between the different cells type forming the BBB,

- fluid shear stress in blood vessels,

- basal membrane (BM) for macromolecule penetration15.

In order to replicate the internal environment, many different design methods and materials are used. The design of in vitro BBB model can be divided in three main categories: sandwich, parallel and 3D tubular structure (as reported in Figure 1.5)15.

Figure 1.5 Designs of a microfluidic BBB-on-a-chip. (A) In the Sandwich design the upper vascular channel and the lower brain channel are separated horizontally; (B) The Parallel design is formed connecting the parallel vascular and brain channels through microchannels; (C) In 3D Tubular Structure design, a cylindrical object which represents a vascular channel, is inserted into an hydrogel containing brain cells; (D) The Aided by Hydrogel Design is developed from a parallel design, in which an hydrogel-infused brain channel separates the two vascular channels. Adapted from15.

The “sandwich design” consists of two PDMS channels separated by a porous membrane (polycarbonate, or PDMS membranes with apertures of 0.2-3 μm)15.

Pericytes, glial cells and neurons are first seeded in the upper channel to form the brain environment, incubated overnight, and then vascular endothelial cells are seeded on the lower channel to form the capillary lumen15. The status of cell growth can be easily detected by the color of the discharged medium at first sight15. Thanks to the transparency of the intermediate porous membrane, the difficulty of obtaining high-resolution images decreases15. Moreover, direct staining with fluorescent markers can be performed for the microscopic observation15. The “parallel design” is a planar parallel design, where two channels are separated horizontally by PDMS to culture glial cell on one side and vascular endothelial cells on the other15. The blood channel and the brain channel communicate through pillars spaced 3 μm apart or pipes with a diameter of 3 μm attached to the bottom surface15. These two designs are replicable thanks to the progresses in lithography, but with the second one, the imaging capabilities of optical detection are improved with respect to the first one15. The disadvantage of these designs is that PDMS channels are characterized by all rectangular sections, causing non-uniform medium flow and a non-uniform cells distribution in the channels15. The last model is the “3D tubular structure design” that better simulates the natural structure and function of BBB. It consists of cylindrical channels in which ECs are cultured, surrounded by brain cells seeded in hydrogel15. Hydrogels are suitable for any type of cells, as their characteristics, such as biocompatibility, permeability of molecules and degradation rate, can be significantly modified by changing its physical or chemical properties16. For example, the porosity of hydrogels varies because of material type, composition of polymers, and the degree of crosslinking16. Collagen gel is commonly used to form ECM barriers in BBB devices. The ECM hydrogels are frequently used as the brain side materials as they have similar properties to the basal membrane in the in vivo BBB 16 . The main disadvantage is that although it solves the problem of shear force distribution, it cannot generate branch network15. In addition, there is a method of self-assembling microvessels in hydrogels, which stimulates the structure and function of BBB15.

## 1.4.2 Cell type used to model BBB on chip

The research on physiology and pathology of the BBB represents a great scientific interest and it leads to the development of several in vitro models14.

### 1.4.2.1 Non-cerebral cells origin

Many publications attest that different types of epithelial or endothelial cells of non-cerebral origin can be used for the study of BBB’s function14. For this purpose, MDCK (Madin-Darby canine kidney) cells, which have good paracellular permeability, are widely used14. Unfortunately, there are huge differences between MDCK and brain endothelial cells, such as in brain penetration of drugs14. Despite Human umbilical endothelial cells (HUVECs) are not of cerebral origin, they are a human cell line, and they are used for BBB’s models14. Generally, these models might be suitable to address some specific questions of BBB function, in fact the large number of BBB in vitro models suggests that there is no “perfect” model, so depending on the specific investigation one or the other model can be more advantageous14.

### 1.4.2.3 Cerebral cells

In the field of biological research two types of cell are used: primary cultures and cell lines.

Primary cultures are obtained from isolating cells from a tissue which can be a lab animal or a human patient (healthy or sick)14. For primary culture, the availability and the reproduction rate of a specific cell type significantly impacts the cost to operate and run the platform13. Nowadays primary cultures of endothelial cells isolated from a broad spectrum of mammals are in use14. The most widely used are brain endothelial cells of rat, mouse, pig and bovine14. The rodent model is the best characterized one and it has a lot of advantages like the availability of experimental animals and the possibility to use transgenic animals in the case of mouse models14 Generally, the more the cellular environment is similar to the physiological characteristics and responses of the BBB, the more reliable the model will be13. Therefore, the use of human primary cells seems to be the most suitable, as it provides the closest approximation to brain microvascular endothelium, but these cells have a lot of complications and disadvantages13:

- high costs for the user;

- limited availability: it depends on a clinical counterpart;

- time-consuming isolation and purification process from the brain tissue of the patient;

- specific technical skills are required for the isolation and purification process to eliminate contaminants such as fibroblasts, pericytes and others;

- very low yield of the isolation process;

- prolonged period of time for culture primary cells;

- rapid differentiation in culture, limiting the amount of useful passages;

- susceptibility to internal and external contamination13.

So, the only advantage of primary cells is the retention of pathological properties, related to brain disorders, of the tissue removed surgically, as cells are in someway patient-specific13.

Generally, the simplest and most feasible in vitro BBB model consists of a monolayer of BECs seeded on a semi-permeable support, under static culture conditions13. However, this model shows a lack of stimuli afforded by the signals of the neighbors Astrocytes and Pericytes13. So, in vitro multicellular models of BECs co-cultured with Astrocytes and Pericytes are widely used since they play a crucial role in the paracellular tightness of the BBB and in modulating BECs’ functions13. Recently developed multicellular BBB models have incorporated BECs with NSCs (neural stem cells), astrocytes, pericytes and MSCs (mesenchymal stem cells), and so the shift to multicellular BBB models with one or more additional NVU cell types has greatly expanded their potential beyond drug permeability screening13.

### 1.4.2.4 Immortalized cell lines

Given the many resources required to obtain primary brain endothelial cells, several cell lines, originating from mice and pig, which can be used up to 35-40 passages, have been developed14. These cells are relatively reliable, easily accessible, affordable and culture preparation time and costs are reduced13. Immortalized brain microvascular ECs come from different species including rodents, humans, and porcine13. However, there are several drawbacks to consider when using immortalized cell lines such as altered expression of characteristic BBB endothelial features, including tight junctional (TJ) proteins, efflux transporters and physiological behaviors such as unresponsiveness to glia or Pericytes stimuli when used in co-culture settings (demonstrated by a low trans-endothelial electrical resistance - TEER)13.

# 1.5 Glioblastoma cancer

From statistical data it emerges that approximately half of all malignant primary brain tumors are glioblastoma (46.1%), which can be distinguished in the two magnetic resonance images on Figure 1.617. However, compared to other cancer entities, the annual incidence is low (3.2 per 100,000)17. The median age at diagnosis is 64 and generally survival rates decrease with age17. Unfortunately, despite the better prognosis of younger glioblastoma patients, no curative therapies exist for either old or young patients17.

The brain composition of hundreds of different cell types poses the question of which is the cell of origin of glioblastoma17. Since DNA is more sensitive to mutational stress during replication, a small group of brain cells termed neural stem and progenitor cells (NSPC) has been suggested as the most likely responsible of glioblastoma17. Actually, these cells retain the ability to replicate as they play a major role in learning and memory as well as tissue repair after injury17.

The blood brain barrier (BBB), which insulates the brain microenvironment from the blood, is strongly linked to the drug-resistant phenotype of glioblastoma, being a major physical and physiological hurdle for the delivery of chemotherapy agents into the brain18.

Under physiological conditions, BBB is able to respond to a variety of regulatory signals from both the blood and brain sides, but its activities can be significantly disturbed under pathological conditions18. Glioblastoma development alters BBB integrity, however surgical and radiographic information have evidenced that all glioblastoma patients present brain regions with an intact BBB, sufficient to prevent drug distribution to tumor cells18. In fact, as previously mentioned, the BBB restricts the diffusion from the bloodstream into the brain parenchyma of 100% of large-molecule and 98% of small-molecule drugs, by means of tight junctions between capillary endothelial cells18.

(a)

(b)

Figure 1.6 Neuroimaging features of glioblastoma. Magnetic resonance imaging (MRI), (a) T1-weighted gadolinium-enhanced, (b) fluid-attenuated inversion recovery (FLAIR). Adapted from17.

This evidence suggests that research progresses on the delivery of therapeutic agents across the BBB are essential to making significant improvements in glioblastoma treatment18.

# 1.6 3D Tumor Spheroid to cancer disease modelling

Up to now, flat 2D cell cultures have always been the most used in vitro method for therapeutic screening because of their simplicity, reproducibility, and low cost, but are unable to reproduce the properties of in vivo solid tumors such as drug-resistance19. Consequently, many ineffective drugs proceed to the in vivo assays, causing the overuse of animals in experimentation, and increasing time and costs of the drug discovery process19. Recently, in vitro 3D cell culture methodologies are investigated as a viable intermediate step between standard 2D cancer cell cultures and in vivo animal experimentation19. So, three‐dimensional spheroids are successfully used for the development of new anticancer agents as they are able to closely mimic the main features of human solid tumors, namely their structural organization, cellular layered assembling, hypoxia, and nutrient gradients19.

3D Spheroids can be produced by seeding cells on artificial matrices called solid scaffolds which can be hydrogels, fibers, or beads which can be produced with different porosities and mechanical characteristics, in order to mimic the extracellular matrix (ECM) of in vivo tumors20. When cells are seeded on these artificial structures, they attach to the matrix and then migrate to the interior of the structure, forming the microtissue in the interstices of the matrix20. However, in some cases it is difficult to extract and collect all the cells from the 3D scaffolds and this compromises spheroids’ analysis20. To surpass these limitations associated with scaffold-based methods, spheroids can be formed by using scaffold-free approaches, where microtissues formation occurs by forcing cells to self-aggregate20.

So, spheroids are cell aggregates (Figure 1.7), self-assembling in an environment which prevents attachment to a flat surface21. Spheroid formulation is possible because of membrane proteins (integrins) and extracellular matrix proteins21. During spheroids formation three steps could be defined21:

- dispersed cells aggregate, due to the binding between long-chain ECM and cell-surface integrin, leading to upregulated cadherin expression,

- cadherin accumulates on the surface of cell membrane,

Figure 1.7 Formation of 3D spheroids. Adapted from21.

the hemophilic cadherin–cadherin binding (in presence of Ca2+) between neighboring identical cells allows to tighten connections between cells, obtaining spheroids.

During this process, cells also produce and organize their own 3D ECM and therefore the matrix is completely endogenous20. The scaffold-free methods include agitation-based techniques, the hanging drop technique, the LOT (Liquid Overlay Technique) and micro-based techniques20.

Large spheroids (> 500 μm in diameter) are capable of mimicking various properties of human solid tumors with sizes from 0.5 to 1 mm3. In particular, the oxygen delivery to the cells located in the inner regions of the in vivo tumor mass is frequently reduced because of the increased oxygen diffusion distances between vasculature and cells19. Consequently, a gradient of oxygen is established as reported in Figure 1.8, and the main causes are the following19:

- cells in the external layer of the tumor have a high access to oxygen compared to those within the tumor which are in hypoxia;

- the oxygen diffusion distance increases with the spheroid growth and consequently, the oxygen diffusion from the medium to the core of spheroid is reduced;

- during spheroids growth, the proliferation of the cells in the external region of the spheroid leads to a higher consumption of oxygen by these cells.

The hypoxic environment within spheroids’ core can also support the development of cancer cells resistance to therapeutics, as some drug’s compounds need oxygen to induce a significative anticancer effect19. On top of that, oxygen-deprived cells originate an acidic microenvironment in human solid tumors (pH of 6.5–7.2) due to the increased production of lactate19. This characteristic is present also in spheroids (Figure 1.8), showing lower pH values in the deepest regions and consequently leading to an impact on drugs efficiency 19.

Figure 1.8 Lactate production and oxygen availability within a single 3D tumor spheroid. Adapted from19.

# 1.7 Aim of the thesis

Glioblastoma, a malignant brain tumor which originates from astrocytes, represents the most common and aggressive form of brain tumor in adults. It is highly invasive, fast-growing and it is characterized by the resistance to standard treatments like chemotherapy and radiation, in fact, due to its aggressive nature, the survival time after diagnosis is relatively short.

The development of a microfluidic device to reproduce a BBB in vitro model is an efficient way to study the paracellular permeability, which is linked to the effectiveness of drug delivery in the medical field. Once a viable culture of NVU cells is obtained into the microchannels of this microfluidic platform, and 3D glioblastoma spheroids are seeded into the microwells, the objective of this work is to administer a specific therapy and test if the tumor target is reached.

Therefore, the aim of this study is to create a BBB-on-chip with permeability and Trans Endothelial Electrical Resistance (TEER) values comparable to those measured in vivo and to administer engineered monocytes to verify whether they can penetrate the barrier dimensionally and reach glioblastoma spheroids also seeded inside the device below the BBB.

# Chapter 2

# Materials and methods

This Second Chapter reports the design and the techniques to fabricate the BBB-on-chip microfluidic model. It also details the design of experiments for its validation in terms of permeability and TEER. The protocols for the biological validation of the models are reported, including all the experiments for the formation and characterization of the Glioblastoma 3D spheroids.

# 2.1 Design and production of the microfluidic platform

In this section will be described the procedure of soft lithography fabrication for the BBB-on-chip master mold. After that, the protocols to obtain the PDMS microfluidic platform through replica molding will be described.

## 2.1.1 Design principle of the BBB-on-chip

The model of the BBB-on-chip designed in this work is presented in Figure 2.1. The design chosen for the BBB device is the horizontal type, modeled in a sandwich arrangement. The platform is composed by three different layers:

  - The apical compartment (marked in red in Figure 2.1(a and b)) which recreates the blood side of the membrane and where the cell medium or drugs will be perfused, resembling the blood flux;

  - The membrane layer (visible in black in Figure 2.1(b)): the membranes selected are commercially available from Sterlitech Corporation. The membranes are polyester (PETE) transparent membrane with pore diameter equal to 3 μm, a thickness of 12 μm and a pore distribution equal to 6*105 pores/cm2. This type of membrane is biocompatible and is ideal for use in blood assays, microscopic analysis, and general filtration. The surface of the PETE membranes is smooth and flat (excellent for particle visibility and quicker analysis) with pores capable of capturing all particles larger than their precise diameters.

  - The basolateral compartment (marked in yellow in Figure 2.1(a and b)) which represents the brain side. In this layer it is also present the Glioblastoma (GBM) area that corresponds to three microwells to insert the 3D spheroids.

Figure 2.1 Top view of the BBB-on-chip where all the compartments are present: the blood side, the brain side and the GBM cancer area; (b) Lateral view of the BBB-on-chip model where it is possible to observe the blood and the brain layer separated by the porous membrane.

## 2.1.2 Photolithography

Figure 2.2 (a) Final geometry of the BBB-on-chip designed with AutoCAD, all the measurements are expressed in mm; (b) The photomask design obtained with Adobe Illustrator.

The design of the photomask is the starting point for fabricating the mold and is essential for obtaining the desired microfluidic structure. The design of the BBB microfluidic device is done using AutoCAD® software as reported in Figure 2.2(a). Subsequently, the file is modified with Adobe Illustrator (Adobe®), enabling the coloring of the CAD geometry in order to obtain the photomask (Figure 2.1(b)). A photomask facilitates the transfer of a specific pattern onto a photoresist layer when exposed to a UV light source.

SU-8 2000 is a high contrast, epoxy-based photoresist, designed for microfluidic applications and it is available in twelve standard viscosities. Film thicknesses of 0.5 to > 200 μm can be achieved with a single coat process. The exposed and subsequently thermally cross-linked portions of the film become insoluble to liquid developers. Commonly SU-8 2000 photoresist is exposed with conventional UV (350 - 400 nm) light, but it may also be exposed with e-beam or x-ray radiation. During exposure, cross-linking proceeds in two steps: the formation of a strong acid during the exposure step, followed by acid-catalyzed, thermally driven epoxy cross-linking during the post exposure bake (PEB) step.

A normal process of photolithography consists of different steps: soft bake (SB), exposure (E), post exposure bake (PEB), the development process and the final thermal step called hard bake to further cross-link the structures.

### 2.1.2.1 Silicon wafer pretreatment and photoresist deposition

A silicon wafer (diameter equal to 4 inches) is cleaned with Acetone (Sigma-Aldrich), Methanol (Sigma-Aldrich) and distilled water in order to promote the photoresist adhesion on the wafer in following steps. The wafer is dried using compressed air and then it is posed on a hot plate at 105° for at least 10 minutes to remove humidity. The wafer at this point is inserted in a spin coater (WS-650-23NPP, Laurell) and secured in the chuck by vacuum generated by a pump (II-3-GD-T4 X, Edwards). At this point one layer of SU8-2100 negative photoresist (Rohm and Haas, Dow Corning) is distributed and spin-coated over the wafer in order to obtain a final thickness of 250 μm. The spin protocol is reported in Table 2.1.

Table 2.1 Combination of time, speed and acceleration data to obtain a 250 μm thickness layer of SU-8 2100 photoresist.

| t [s] | Speed [rpm] | Acceleration [rpm/s] |
| --- | --- | --- |
| 10 | 500 | 100 |
| 32 | 1000 | 300 |

The wafer is removed from the spin coater and left to rest and degassed for 5 minutes to eliminate the presence of bubbles trapped in the resist. Volatile components of the photoresist evaporate and a partial reticulation gets start.

### 2.1.2.2 Soft Bake and UV exposure

Soft Bake is a heating treatment used mainly to drive the solvent present in the photoresist. Soft Bake also improves the uniformity of the photoresist, its etch resistance and optimize its light absorbance characteristics. This is a critical step in that failure to sufficiently remove the solvent will affect the resist profile, while an excessive baking, will destroy photoactive compound and reduce its sensitivity. The Soft Bake is done placing the wafer on a hot plate at 95°C for 45 minutes as reported

Figure 2.3 Hot level plate used in the Soft bake step. It is placed in clean room under chemical hood.

in Figure 2.3. During the Soft Bake step, convection ovens are not recommended, otherwise a “skin” which inhibits the evolution of solvent may form on the resist, resulting in incomplete drying of the film and/or extended bake times. Before the exposure step is necessary align the photomask with the wafer in order to polymerize the negative photoresist in the areas exposed to UV light. The photomask is placed on the top of the wafer and exposed to UV rays with a power of 350 mJ/cm2 for 120 seconds, as shown in Figure 2.4. Generally, the power and the exposition time depend on the height of photoresist. The time exposition is calculated with the following equation:

|  | t=EexpP, | (2.1) |
| --- | --- | --- |

Figure 2.4 (a) UV light with specific filter; (b) Photomask adherent to the wafer where negative photoresist SU-8 2100 has been deposited before.

where Eexp is equal to 245 mJ/cm2 and P is the power of the UV light, equal to 5 mW/cm2.

### 2.1.2.3 Post Exposure Bake, Development and Hard Bake

A Post Exposure Bake (PEB) is performed on the hot plate at 95 °C for 15 minutes. This step takes place directly after exposure. After some minute of PEB, an image of the geometry impressed should be visible over the wafer. After the PEB step, the wafer is developed in a solvent, propylene glycol monomethyl ether acetate (Sigma-Aldrich) to dissolve not cross-linked polymer. A glass trough is filled with solvent and the wafer is immerged into it, then the glass trough is placed over a rocking shaker for about 17 minutes. The developed image is rinsed with propylene glycol monomethyl ether acetate) and then with Isopropyl Alcohol (IPA, Sigma-Aldrich). Afterwards the surface is dried with compressed air. If a white film is produced during IPA rinse, it indicates the underdevelopment of the unexposed photoresist. It will be necessary to simply re-immerse the substrate with additional SU-8 developer to remove the white film and complete the development process and then repeat the IPA rinse. Finally, the ‘hard bake’ step or ‘final cure’ is done. SU-8 2000 has good mechanical properties and to maintain them, a hard bake step is incorporated into the procedure. This is required only if the final device has to be subject to thermal processing during regular operation: SU-8 2000 is a thermal resin so its properties can continue to change when exposed to a higher temperature than previously encountered. For this reason, a final bake temperature 10°C higher than the maximum expected device operating temperature is recommended. Depending on the degree of cure required, a bake temperature in the range of 150°C to 250°C and for a time between 5 and 30 minutes is typically used. In this case, the wafer mold was placed on the hot plate at 100°C for 2 hours. Then, the master is left onto the hot plate till ambient temperature, in order to avoid thermic shock.

## 2.1.3 PDMS Replica molding

Figure 2.5 (a) Kit provided by Dow Corining Sylgard 184 used for the PDMS preparation; (b) Desiccator used in the laboratory used to eliminate the air bubbles trapped in the PDMS solution.

At first the wafers of device mold are treated with chlorotrimethylsilane (Sigma-Aldrich) vapor for 60 minutes in order to make easier the extraction of the PDMS from the mold after the curing. This step is called “silanization” and improves the durability of the mold.

PDMS is prepared using the kit provided by Dow Corning Sylgard 184, shown in Figure 2.5 (a). It consists of a silicone base and a curing agent that are mixed in a weight ratio of 10:1 ratio. The solution is then stirred for at least 2 minutes in order to reach homogeneity. Since a lot of air bubbles remain entrapped within the viscous solution during stirring, the solution is put in a desiccator for almost 30 minutes, to eliminate them. Some grams of PDMS are poured on the master and then it is degassed again to remove completely bubbles. The PDMS in the glass is covered with a piece of aluminum foil and stored in the fridge at 4 °C up to 24 hours.

The PDMS is curing for 60 min and 75°C in the oven. Once PDMS is completely reticulated, the layer is slowly removed from the wafer. Inlets, outlets (1 mm of diameter) and central wells (2 mm of diameter) are punched with biopsy punch.

## 2.1.4 Plasma bonding

The porous PETE transparent membrane (Sterlitech Corporation) with pore diameter equal to 3 μm is cut and placed in correspondence with the horizontal microchannel of the bottom PDMS layer of the microfluidic device. Subsequently the bottom and the top PDMS layer are irreversibly attached together using the Plasma Cleaner, as reported in Figure 2.6. Plasma treatment has the capability to form covalent bond between the two PDMS layers. As the final step, the device is attached to a glass slide that serves as a support, especially during microscope analysis.

Figure 2.6 Harrick Plasma Cleaner used in Biamet laboratory. Adapted from27.

Plasma is an ionized gas atmosphere with ions and electrons but globally neutral. The surface to be activated is put inside the plasma facing up. Vacuum pump is activated for 5 minutes and then plasma is activated for 1 minute, regulating the intensity through the vent valve. In this way the atmosphere is enriched with oxygen. The cloud has to be pink and not violet.

Afterwards the plasma chamber is gradually brought to atmospheric pressure, and the two layers are put in contact applying a little pressure and paying attention not to touch the activated surfaces. The device is put in the oven at 65 °C for 15 min to seal the bond. The same procedure is applied to attack the complete microdevice over the glass slide.

# 2.2 Leakage test of the BBB-on-chip

The final BBB-on-chip can be validated with colorants to ensure the correct sealing and operation. Thanks to the optical properties of PDMS the platform can be easily observed with the microscope. When the two layers of the device have been assembled, a seal test is necessary to verify if the plasma treatment has provided enough resistance to the pressure flow.  The colorants validation is done using a food coloring solution prepared by mixing it with water. A syringe pump is used to infuse the solution inside the micro-bioreactor. The pump PHD Ultra (Harvard Apparatus) is composed by two elements: a central body with a display used to set up flow rates and a mechanical multi-rack unit where syringes can be placed. The experiment procedure consists of the following steps:

- The device is filled using the inlets of the platform with physiological water to remove all the bubbles of air trapped inside it (this step is called “debubbling”);

- 5 mL syringes (NYPRO) are filled with the colorants mixtures: a 21G steel dispensing tip (DRIFTON) is inserted inside a microtube (inner diameter 0.5 mm, outer diameter 1.5 mm, TYGON® TUBING) and then connected to the syringe;

- Two syringes, one with a red colorant and one with a yellow colorant are placed on the infuse side of the pump;

- A flow rate of 5 μL/min is set, the pump is started and once the fluid has completely filled the tube the syringes are connected to the platform.

# 2.3 Cell Cultures

This section explains in detail the protocols followed to correctly maintain the different cell lines used to create the BBB model.

## 2.3.1 Thawing and maintenance of cells

Astrocytes, Pericytes and HUVEC are provided by Innoprot. These cells are stored frozen in liquid nitrogen at a temperature equal to -80 °C and conserved in this way for several months. Cells stay into a vial in a solution of CryoStor CS10 (Biolife Solutions) which is necessary to prevent their increasing of volume during the change in phase from liquid to solid. Dilution and centrifugation of cells after thawing are not recommended, so cells are directly seeded into a coated T-75 flask with about 10 mL of fresh medium. The flask is maintained in the incubator at 37 °C. Medium is changed every 2-3 days till the whole surface of the flask is covered by cells: this is called confluent condition and splitting (subculture) is needed. So, by dividing the total number of cells on two or more flasks, cells will have more space to grow and reproduce again.

### 2.3.1.1 Astrocytes

Astrocytes (Immortalized Human Astrocytes) constitute the main part of the cells in the mammalian brain, they are intimately associated with neurons and also linked to the blood vessels supplying the central nervous system. They provide structural, trophic, and metabolic support to neurons and modulate synaptic activity. Astrocytes (P10251‐IM, Innoprot) have been obtained immortalizing Human Cortical Astrocytes with Lenti‐SV40 Lentivirus, so their genome has been modified in order to let themselves to reproduce indefinitely. In fact, primary cells go into senescence after the 5th passage, while the SV40‐tranduced cells go beyond 30 passages.

The number of cells stored in liquid nitrogen is higher than 1*106 in cryopreserved vials. In general, cryopreserved cells are very delicate, so the vial is thawed in a 37 °C water bath and turned into culture with minimal handling. The set up of the culture includes the following passages:

  - Under a biological hood, a poly‐L‐lysine solution prepared in distilled water with a final concentration equal to 2 μg/cm2 is added to the flask and left overnight in incubator. The specific medium of astrocytes is necessary, so the complete media include: 500 mL of Astrocyte basal medium, 10 mL of fetal bovine serum (FBS), 5 mL of Astrocyte Growth supplement (ACGS) and 5 mL of penicillin/streptomycin solution (P/S).

  - The poly‐L‐lysine‐coated vessel is rinsed twice with sterile water and then 15 mL of medium are added.

  - To thaw the cryopreserved cells, the frozen vial is placed in a 37 °C water bath and the content of the vial is resuspended into the poly‐L‐lysine‐coated culture vessel (seeding density of 5000 cells/cm2 is recommended).

To maintain the culture, it is necessary to refresh medium the next morning after establishing the culture from cryopreserved cells. Afterwards the medium is changed every three days, until the culture is approximately 70% confluent. Once the culture reaches 70% confluency, the medium is changed every other day until the culture is approximately 90% confluent. When the flask reaches confluency, a subculture is established by splitting the cell culture with the following protocol.

    - Poly‐L‐lysine‐coated culture vessels is prepared (2 μg/cm2) one day before.

    - Medium, trypsin EDTA (Corning), and PBS (Gibco) are warmed in the water bath. Trypsin is an enzyme used to detach cells from the flask’s surface and between each other. This protease is inhibited by Calcium and Magnesium, ions naturally present in the culture medium. PBS is a saline solution without these ions, so it promotes trypsin action.

    - The medium in the flask (8 mL) is collected into a 50 mL falcon.

    - The cells in the flask are rinsed with PBS.

    - 8 mL of PBS and 2 mL of trypsin are added into the T-75 flask.

    - The flask is incubated at 37°C for 3 or 4 minutes. To verify if the cells have been detached, an optical microscope is used as they change their morphology, becoming spherical.

    - The trypsin and PBS solution with cells is transferred from the flask to the 50 mL falcon with the medium which inactivates trypsin.

  - The flask is examined under the microscope: now there should be less than 5% of the initial number of cells.

  - The 50 mL falcon is centrifuged at 1000 rpm for 5 minutes with soft deceleration.

  - The medium is withdrawn, paying attention not to take out the pellet of cells.

  - Cell pellet is resuspended at the bottom of the falcon in the amount of culture medium needed for the future experiment.

  - Cells are plated in a new poly‐L‐lysine‐coated culture vessel with the recommended cell density.

### 2.3.1.2 Pericytes

Pericytes are contractile (muscle‐like) cells that cover the abluminal surface of microvessels. They are most abundant on venules and are common on capillaries. Pericytes have four major functional roles:

- central nervous system microvascular contractility;

- regulation of endothelial cell activity;

- macrophage activity;

- role in the transport across the blood‐brain barrier and the regulation of vascular permeability.

Human Brain Vascular Pericytes (HBVP) are cryopreserved after the purification, delivered frozen in liquid nitrogen (5*105 cells in 1 ml vial) and are guaranteed to further expand for 10 population doublings. The set up and the maintenance for the culture is the same of astrocytes. So, to summarize it is necessary to prepare a poly‐L‐lysine coated flask and then seed pericytes. To maintain the culture, it is important to keep the flask in the incubator and change medium after about 3 days. When confluence is reached, a subculture is set up, following the same protocol described in Section 2.3.1.1.

Pericyte Medium (ECM) contains 500 mL of Basal Medium, 10 mL of fetal Bovine Serum (FBS), 5 mL of Pericyte Growth Supplement (PGS) and 5 mL of penicillin/streptomycin solution (P/S solution).

### 2.3.1.3 HUVEC

The vascular endothelial cells maintain vascular homeostasis, secrete activators and inhibitors of both the coagulation system and the fibrinolysis system. These cells also synthesize mediators that influence the adhesion and aggregation of blood platelets. Additionally endothelial cells release molecules that control cell proliferation and modulate vessel wall tone. Human umbilical vein endothelial cells (HUVEC) are the most used for endothelial processes studies in vitro.

RED-TTFFLUOR HUVEC are red fluorescent HUVEC cells which express the red fluorescent protein gene sequences as free cytoplasmatic protein. The storage is done in liquid nitrogen and the number of cryopreserved cells is higher than 106. Endothelial cell medium is recommended to maintain the culture. To set up the culture the following passages are requested:

1. A fibronectin coated flask (2 μg/cm2 in a T-75 flask) is prepared. Fibronectin is a protein which promotes cell attachment and growth, so 10 mL of sterile DPBS (Dulbecco’s phosphate buffered saline) and then 150 μL of fibronectin solution (1 mg/mL) are added to a T-75 flask. The flask is left in incubator overnight.

2. The fibronectin solution is rinsed away with PBS and 10 mL of complete medium are added. The flask is left in incubator.

3. To thaw cells, the vial is placed in a 37 °C waterbath and it is gently rotated until the content is completely thawed. The vial is removed from the waterbath, rinsed with 70% ethanol and transferred in the hood. 1 mL pipette is used to re-suspend the content of the vial.

4. The content of the vial is dispensed into the fibronectin coated flask full of medium (a seeding density of 5000 cells/cm2 is recommended). The flask is stored into the incubator and the culture is not disturbed for at least 16 hours. The growth medium is changed regularly.

Generally, a healthy culture will display cobblestone or spindle shaped morphology. The cell number will be double after two to three days in culture. The maintenance of culture includes changing medium the morning after the thawing and then every three days thereafter, until the culture is approximately 70% confluent. As for all types of cells, once the culture reaches 70% confluence, change medium every other day until the culture is approximately 90% confluent. At this point a subculture is set, following the steps listed below.

1. Fibronectin coated flasks are prepared one day before subculture.

2. The exhausted medium is collected into a 50 mL falcon.

3. Medium and trypsin are warmed at 37 °C.

4. Cells are rinsed with 10 mL PBS and then 2 mL of trypsin are added into the flask that is gently rocked to make sure cells are covered by trypsin solution; the flask is incubated at 37 °C for 3 minutes or until cells are completely rounded up. The flask is monitored under optical microscope to make sure all cells are detached.

5.Trypsin solution with detached cells are transferred from the flask to the 50 mL Falcon. The flask is examined under inverted microscope to make sure all cells are removed. There should be less than 5%.

6. The 50 mL Falcon is centrifuged at 1000 rpm for 5 min with soft deceleration; the exhausted medium is removed and cells’ pellet is re-suspended in fresh medium.

7. Cells are counted and plated in a new fibronectin coated flask, with cell density as recommended.

## 2.3.2 Cell count

The tool used for cell counting procedure is a hemocytometer (also called Bürker chamber). An example of this device is reported in 2.7(a): it is formed by a rectangular microscope slide with a rectangular indentation that creates a chamber. This latter is engraved with a laser-etched grid of perpendicular lines. The area enclosed by the lines is known, as well as the volume of the chamber. Therefore, it is possible to count the number of cells present in a specific volume of fluid, and so the total number of cells.

(a)

(b)

Figure 2.7 (a) Burker Chamber; (b) Trypan Blue used to stain dead cells.

More precisely, the Bürker chamber is formed by two 3 x 3 mm big square, with a depth of 0.1 mm. Each of these is divided in 9 squares (1 mm side), each one further divided in 16 smaller squares (0.2 mm side). Moreover, during this step it is also used Trypan Blue (reported in Figure 2.7(b)). The last one is a selective dye: membranes of live cells are impermeable, while the dead ones turn blue (distinguishable at microscope).

The procedure to count the total number of cells is the following one:

- After trypsinization, 10 μL of the cell suspension are collected on one of the corner of the Burker chamber. Then, 10 μL of Trypan Blue (Invitrogen) are added, obtaining a solution of 20 μL of Trypan Blue with cells;

- 10 μL of this solution are gently pipette between the chamber and the glass slide;

- The microscope (with a 10x magnification) is used to count cells within and at least 5 squares are considered;

- The number of total live cells in the solution is obtained by the 2.8 formula:

|  | Celltot=M*2*104*Volumetotal, | (2.8) |
| --- | --- | --- |

where M is the average value of cells counted in the 5 squares of the Burker chamber; 2 is the dilution factor for the Trypan Blue, 104 is a corrective factor related to the geometry of the hemocytometer and Volumetotal is the solution of medium and cells (mL) within the falcon.

# 2.4 BBB-on-chip microfluidic model: cells seeding

The first step is to sterilize the microdevice using UV light under a biological hood for almost 1 hour. To improve cell adhesion, a coating on the membrane layer is necessary, in particular Fibronectin is used for the upper part of the membrane, while poly-L-lysine for the bottom side. The vascular and the brain cells are seeded in two different days, so PLL and FN are not applied at the same time. The protein concentration is the one used for the coating protocols, described in Section 2.3.1. Microdevices are incubated overnight and the day after the coating is washed out with PBS. As final step the specific medium is used to fill the microchannels to prepare a suitable environment for cells. For first astrocytes and pericytes are seeded on the membrane in the basolateral compartment through Inlet 1 (see Figure 2.9(a)). The splitting procedure is the same described in Section 2.3.1 and the cells are diluted in fresh media to reach a seeding density of 1*106 cells/mL where 2/3 of cells are Astrocytes and 1/3 are Pericytes in the total volume. In this case the cells are diluted only in the media of Astrocytes that is similar to that of Pericytes. After the seeding all the inlet/outlet of the device are closed with sterile iron wires and the platform is flipped, and incubated overnight, leading cells to attach on the membrane side. Once cells have attached to the membrane (after 4-5 hours from seeding), there are two methods to supply fresh medium: static and dynamic. In the static method, shown in Figure 2.9(a), four pipette tips are used to close the inlet/outlet. Medium is aspirated from its container with one pipette and then the tip is stuck into the inlet hole punched. Now, applying a little pressure on the tip with a finger, medium is released and it fills the chamber. However, this method is not so efficient since often leakages appear and/or medium does not well flow through the microchannel, influencing the success of the experiment.

Figure 2.8 (a) Static medium supply of the BBB-on-chip; (b) Set up for the dynamic perfusion of cell culture media inside the device using a syringe pump.

Figure 2.9 Structure of the BBB model inside the device: in the upper side of the membrane HUVEC are seeded to recreate the blood compartment while in the lower side Astrocyte and Pericyte are seeded to mimic the brain compartment.

The simplest and most efficient method is to use a syringe pump to supply constantly medium to the cell culture seeded inside the microfluidic device. A syringe is filled with 3 mL of Astrocyte medium and connected to Inlet 2 of the device. A flow rate equal to 20 μL/hr is set, which fills the top horizontal channel, passes through the membrane and reaches the bottom channel. All the other holes, except the 3 central microwells are accurately closed with iron wires. After 24 hours from Astrocytes and Pericytes seeding, endothelial cells (HUVEC) are seeded from the Inlet 2 in order to cover the upper part of the membrane to recreate the structure of the BBB model as reported in Figure 2.10.

After 5 hours from HUVECs seeding, a solution made by 50% of Astrocyte media and 50% of HUVEC media is prepared, and the syringes are filled and connected to the device to supply fresh media with the pump. The devices are placed in the incubator for almost 7 days.

# 2.5 GelMA 3D model to mimic the BBB

Figure 2.10 Model of the 3D GelMA construct with HUVEC, Pericytes and astrocytes seeded to form the three different layers starting from the top, aiming to mimic the in vivo organization of the neurovascular unit.

GelMA (Gelatin methacryloyl) is one of the most popular biomaterials used in tissue engineering22. It is a freeze-dried product derivated from gelatine of animal origin and functionalized with methacrylate groups22. The introduction of methacryloyl substituent groups leads to a photo-crosslinking using lithium phenyl-2,4,6-trimethylbenzoylphosphinate (LAP) photoinitiator and UV light22. This polymerization takes place in aqueous environments, at room temperature and neutral pH22. One of the starting ideas is to realize the BBB barrier by using GelMA hydrogel to mimic the 3D environment22. The model involves obtaining 3 different layers with Astrocytes, Pericytes and endothelial cells seeded from bottom to top22. The total thickness of the structure must be equal to 400 µm, as represented in Figure 2.8, which is similar to the one present in the human body22.

GelMA is stored in the freezer into a sterile falcon and its preparation includes the following steps:

- In order to know the weight of the piece of lyophilized GelMA considered and to hold sterility, for first a 50 mL empty falcon is weighted.  The container is opened under hood and a piece of GelMA is cut with a sterile blade and transferred into the 50 mL empty falcon. Now the falcon with GelMA is weighted. The difference between the two weights is calculated to know exactly the weight of the product taken for the experiment.

- To use GelMA it is necessary to dissolve the dehydrated product into a solution of PBS and 0.1 % of Lithium phenyl-2,4,6-trimethylbenzoylphosphinate (Laponite, Sigma-Aldrich) that is a water soluble, cytocompatible, Type I photoinitiator used in the polymerization of GelMA hydrogels22. The GelMA concentration to recreate the BBB model is selected equal to 8% and the total weight is calculated with the following proportion22:

|  | gGelMA:8%=X:100%, | (2.2) |
| --- | --- | --- |

where: gGelMA is the total weight of GelMA in gr, X are the mL of PBS with 0.1% LAP (weight/volume) that is decided following the specific experimental requirement.

- After the adding of the GelMA lyophilized in the solution of PBS with the 0.1% of Laponite, the falcon is put on the vortex to mix well the solution and put into the thermostatic bath for 30 minutes22. Once GelMA is dissolved, the falcon is centrifuged at 3000 rpm for 5 minutes22. Afterwards the solution must be filtered with a 0.22 filter to sterilize it22.

Now GelMa is ready to use, to recreate the BBB 3D model. PDMS empty disks are realized following the protocol to prepare PDMS, described in Section 2.1.2. Three different GelMA hydrogel solutions are prepared (as the BBB model is composed by three different cell lines) with a precise amount of medium and cells in such a way as to obtain a concentration of 1*106 cells/mL of hydrogel and with a volumetric hydrogel/medium ratio below 1:25. Once the hydrogel solution is produced, cells are added with the medium before subjecting it to UV exposure. Then, 100 μL of the new mixture are taken with a micropipette to fill the PDMS empty disks. Afterwards they are subjected to the UV lamp, realizing three-dimensional structures with encapsulated cells. This procedure is done starting with the Astrocyte GelMA.

# 2.6 Biological validation of the BBB model

In the following paragraphs, the experimental protocols conducted to validate and confirm the formation of the BBB inside the microfluidic device and in the 3D GelMA construct are described.

## 2.6.1 Live & Dead assay

Vitality test is carried out by using a fluorescence microscope, which is an optical microscope that exploits the phenomena of fluorescence induced by fluorescent markers.

Fluorescence is the property of certain substances to re-emit (in most cases at a longer wavelength and, therefore, lower energy) the received electromagnetic radiation. The marker is a fluorochrome, which is a dye which absorbs high frequency ultraviolet waves and emits electromagnetic waves in the visible range. The excitation wavelength for Hoechst is done by ultraviolet light around 350 nm, while the emission wavelength is 461 nm, resulting in a cyan blue color when observed under the microscope. The Calcein-AM marker has an excitation wavelength of 495 nm and an emission wavelength of 515 nm, appearing green. Propidium is excited by wavelengths of 493 nm and emits at a maximum of 636 nm, which is in the red range.

For the vitality tests conducted on BBB-on-chip, Calcein-AM was used to highlight the cytoplasm of living cells, Hoechst to label the nucleus of all cells and Propidium to mark the nuclei of all dead cells. Hoechst and Propidium are stored at 4 °C, while Calcein is stored at -20 °C. All these fluorescent markers are photosensitive so it is important to turn off lights during the operations.

For the preparation of 1 mL of staining solution, Calcein-AM (Sigma-Aldrich) is diluted 1:1000, Hoechst (Sigma-Aldrich) is diluted 1:500 and Propidium (Sigma-Aldrich) is diluted 1:250 in PBS (or pure culture medium without FBS).

The protocol of the Live & Dead assay includes the following steps:

- The medium present inside the devices is removed and all the channels (apical and basolateral) are washed well with PBS to eliminate any traces of serum-containing medium. This is fundamental as FBS (fetal bovine serum, a supplement present in the culture medium) interferes with the staining solution. For the GelMA BBB model the scaffolds are placed in a 24 multiwell and covered with PBS for 30 minutes to wash the media present inside;

- The staining solution is prepared as described above and the channels of the device are completely filled with the solution. For the GelMA scaffolds, the PBS are discharged and the staining solution is added in the 24 multiwell to cover all the GelMA BBB sample.

- The sample (microfluidic device and GelMA constructs) are placed in an incubator for at least 30 minutes.

- Subsequently, specific wavelengths λ are used to excite the fluorescent markers. The three markers are excited by specific LED: DAPI for Hoechst, GFP for Calcein and Texas Red for Propidium.

## 2.6.2 Fixation protocol using PFA

In the fixation step utilizing 4% paraformaldehyde (PFA, Sigma-Aldrich) for cell samples, the goal is to preserve cellular structures and biomolecules while halting any biological processes. Paraformaldehyde, a cross-linking agent, penetrates the cell membrane and binds to proteins, DNA, and other cellular components, effectively immobilizing them. This concentration of PFA is chosen for its ability to efficiently fix cells without causing excessive distortion or damage to delicate structures. First, the microfluidic device is rinsed with PBS. In particular, 3 washes are performed for 5 minutes each. Subsequently, in the case of 2D cell culture as that inside the BBB microfluidic device, cells are fixed with a solution of 4% PFA in PBS for 15 minutes at room temperature. In 3D spheroids time and temperature combination varies, in fact PFA fixation occurs at 4 °C for 1 hour. After this step, 3 washes for 5 minutes each with PBS are done. Following fixation, the cells can be further processed for various applications such as immunostaining and fluorescence microscopy, enabling examination and analysis of cellular morphology and molecular distribution.

### 2.6.2.1 Phalloidin & DAPI

Phalloidin (Proteintech), a phallotoxin produced by death cap mushroom Amanita phalloides, is used in fluorescent microscopy to identify actin filaments, a protein which constitutes the cell's cytoskeleton. DAPI (Life Technologies), short for 4',6-diamidino-2-phenylindole, is a fluorescent stain commonly used in microscopy to label DNA. It binds strongly to the adenine-thymine (AT) rich regions of DNA, emitting blue fluorescence upon excitation with ultraviolet light. This property makes DAPI an invaluable tool for visualizing the cell nucleus and chromatin structure.

The protocol to perform the staining with Phalloidin and DAPI markers includes the previously described fixation in PFA. Subsequently, Phalloidin is diluted in PBS at a ratio of 1:200, while DAPI 1:1000. The solution is then incubated for 40 minutes at 37°C (at least 1 hour for 3D spheroids), followed by 3 final washes with PBS, each lasting 5 minutes.

### 2.6.2.2 Immunofluorescence

Immunofluorescence staining is a technique that uses the specificity of antibodies to their antigen to target fluorescent dyes to specific biomolecule targets within a cell. There are two classes of immunofluorescent techniques, primary (or direct) and secondary (or indirect). The direct technique uses a single, primary antibody, chemically linked with a fluorophore which is a fluorescent chemical compound that can re-emit light upon light excitation. The indirect technique, employed in this experiment, uses two antibodies: the primary antibody specifically binds the target molecule and the secondary antibody, which carries the fluorophore, recognizes the primary antibody and binds it. This provides signal amplification by increasing the number of fluorophore molecules per antigen. Cultured cells in microfluidic platforms, after the procedure of fixing, are incubated with blocking solution containing 5% of Bovine Serum Albumin (BSA, Sigma-Aldrich), 0.1% of Triton X-100 (Sigma-Aldrich) in PBS and are left at room temperature for 1 hour. After 1 hour all the chambers and all the well are washed for three times with a solution of PBS with 0.1% of Triton X-100 (PBST). At this point the primary antibodies are applied. In this experiment we used 2 different primary antibodies:

- CD31 (CD31 Mouse Polyab, Proteintech): known as platelet endothelial cell adhesion molecule-1 (PECAM-1), is a transmembrane glycoprotein primarily expressed on endothelial cells, including those of HUVECs. As a member of the immunoglobulin superfamily, CD31 plays a crucial role in various cellular processes, including endothelial cell junction formation;

- GFAP (GFAP Rabbit PolyAb, Proteintech): is a protein specific to the central nervous system, expressed predominantly in Astrocytes. It is one of the main components of the intermediate filament network in astrocytes, which plays a role in cell migration and motility.

In the table 2.3 the origin of each monoclonal antibody and the dilution chosen are summarized.

Table 2.3 Summary of species of origin and dilution of the primary antibodies used.

|  | Host | Dilution |
| --- | --- | --- |
| CD31 | mouse | 1:200 |
| GFAP | rabbit | 1:250 |

The primary antibodies are diluted in their specific concentration into the blocking and are stored in the fridge at 4°C, keeping dark and humidified conditions within the chamber overnight.

After three PBST washes, the fluorophore-tagged secondary antibody in blocking solution is applied to the samples for 45 minutes at room temperature. Two different solutions of secondary antibodies are prepared each containing secondary antibodies corresponding to the species of the primary antibodies previously used. To detect CD31, the CoraLite488-conjugated Goat Anti-Mouse IgG (Proteintech) is used at a ratio of 1:1000. For GFAP, the CoraLite647-conjugated Goat Anti-Rabbit IgG (Proteintech) is used at a ratio of 1:1000.

Two PBST washes and one with only PBS are done. Then a nuclear stain with DAPI in 1:1000 concentration in PBS is done for 10 minutes, ending with other 3 PBS washes. The microdevice is then filled with mounting media which is composed of 80 % glycerol and 20 % milliQ water. Finally, the stainings are analyzed with a fluorescence microscope (Zeiss Axio Observer confocal microscopy): the microscope has a fluorescence lamp which excite the fluorescent dyes and capture the emitted light through different filters.

# 2.7 Determination of paracellular barrier characteristics in vitro

In the following paragraphs the instruments and methodologies for the permeability and the transendothelial electrical resistance tests are described. These two measurements are fundamental to validate the BBB model on chip.

## 2.7.1 Measurement of Permeability

To experimentally measure permeability of the membrane, three FITC (Fluorescein isothiocyanate) dextrans, with average molecular weight of 40, 70 and 150 KDa, are perfused into the microfluidic platform through a syringe pump at a flow rate equal to 5 μL/min, as represented in Figure 2.11.

Figure 2.11 (a) The syringe pump used to flux fluorescent dextrans into the microfluidic devices; (b) The platform is completely filled with fluorescent dextrans which is collected at the outlet.

(b)

(a)

The fluorescent dye enters through the horizontal channel, passes through the porous membrane, and once it reaches the basolateral compartment, it crosses the three perpendicular channels and fills the three microwells (Figure 2.12). At t0 the stopwatch and the syringe pump are turned on at the same time, so the specific dextran starts to flux. During the operation, photos are taken in sequence at the optical microscope, to monitor the filling of the microwells and to calculate the permeability value.

Figure 2.12 Representative frames of the video used to calculate the permeability: the microwell present in the GB cancer area are filled with dextran perfused from the syringe pump.

Permeability is calculated through the 2.11 mathematical formula23:

|  | P=VbasolateralΔcbasolateralΔtAcapical . | (2.11) |
| --- | --- | --- |

Where:

P = solute permeability cms;

V = volume of the basolateral compartment cm3;

A = surface area of the membrane (cells monolayers) cm2;

C = dextran’s concentration (intensity) in the apical compartment (maximum one) arb. unit;

ΔC = change in solute concentration (intensity) in the basolateral compartment as a function of time arb. unit.

In the mathematical calculation of permeability, the intensity of the dextran is analyzed through ImageJ which measures the mean fluorescence intensity within a particular ROI (Region of Interest) selected. Generally, permeability values in the order of magnitude of 10-6 cm/s for sodium fluorescein are considered good values14.

All the resulting values of permeability are graphed through histograms, using GraphPAD. The paired t-test is also performed, which is a statistical test used to compare the means of two related groups to determine whether there is a statistically significant difference between them. It is typically applied when the same subjects are measured under two different conditions or at two different points in time. The test assesses whether the mean difference between paired observations is significantly different from zero, indicating whether there is a systematic change between the conditions or time points being compared. In particular, the t-test is used to calculate the p-value and the threshold of 5% discerns two situations:

- If p-value ≥ 0.05 there is not a statistically significant difference between the two groups;

- If p-value < 0.05 there is a statistically significant difference between the two groups.

## 2.7.2 Measurement of TEER

The paracellular permeability can be studied also through the measurement of transendothelial electrical resistance (TEER)14. The most widely used ‘classical’ method to determine TEER is based on the culture of endothelial cells on semipermeable filters which define two compartments: the apical upper compartment which can be considered as “blood-side” and the basolateral, lower compartment which is the “brain side”14. The same is done in this work and thus for the measurement of TEER, an electrode is used, one tip is placed in the upper and the other in the lower chamber. Good models have values in the order of magnitude of hundreds 14 and results are expressed in Ohm*cm2.

In this work, the portable Voltohmetro Millicell® ERS-2 (Electrical Resistance System) shown in Figure 2.13 is used, which reliably measures membrane potential and resistance of epithelial cells in culture grown on the microporous membrane.

Figure 2.13 Voltmeter Millicell® ERS-2 (Electrical Resistance System) used into a multiwell to measure TEER in traditional in vitro models. Adapted from28.

The voltmeter is made of a small silver/silver chloride (Ag/AgCl) pellet on each electrode tip which measures the voltage. Due to the small size of the electrodes, the transepithelial voltage can be easily measured not only on multiwells but also on microfluidic platforms.

Fortunately, measurements do not compromise sterility or damage cells on the membrane. The microbioreactor is modified by increasing the thickness of the PDMS layer because the tips of the electrode must be completely submerged by medium.

To measure the resistance, samples are punched in two points (with 4 mm punch diameter): one in correspondence of the membrane and the other near the inlet 1. When cells are seeded, the cup removed by punching the PDMS surface is used to close the chip and maintain sterility. Samples are removed from the incubator at least 30 minutes before the TEER measurement, in order to reach room temperature.

The shorter tip of the electrode has to be completely submerged by cells’ medium, avoiding touch cells over membrane, while the longer tip has to barely touch the bottom of the hole without cells. To get reproducible results it is important to maintain the electrode fixed in perpendicular condition.
The TEER measurement provides values in [Ohm]. For first the resistance of the blank chips is measured (Rblank). Afterwards also the TEER in the chip with cells over membrane is measured (Rsample). To calculate the resistance of the cellular layer, the difference is done as in the 2.12 equation.

|  | Rsample-Rblank = Rcells . | (2.12) |
| --- | --- | --- |

Now this result must be corrected with the value of the surface covered by the cellular layer using the 2.13 equation.

|  | TEER Ohm*cm2= Rcells*A . | (2.13) |
| --- | --- | --- |

where A is simply the area of the membrane.

Some studies have found an important relation between TEER and permeability values: above a TEER value of 120-130 Ohm*cm2, the permeability for sodium fluorescein and FITC-labeled dextrane (4 kDa) does not decrease with the increase of TEER24. This important result indicates that barriers with at least 120-130 Ohm*cm2 can be used for transport experiments, as permeability value has settled at an asymptotic value24.

# 2.8 3D Glioblastoma spheroids experiments

Glioblastoma (GB) spheroids were obtained using Ultra-Low Attachment (ULA) 96-well round-bottom plates (Corning® 7007) as reported in Figure 2.14. In this case, to mimic Glioblastoma cancer spheroids U87 cell line is used. The cell culture media is composed by: DMEM basal media (Gibco) with 10% of Fetal Bovine Serum (Gibco) and 1% of Penicillum/Streptomicina (Corning). Seeding densities of 2000 cells/spheroid were used. A volume of 200 μL/well of trypsinized single-cell suspension at the desired density was dispensed into each well. Then, plates were centrifuged at 1200 rpm for 10 minutes to allow the cells to rapidly aggregate at the bottom of the wells.

(a)

(b)

Figure 2.14 (a) ULA (ultra-low adherent plates); (b) Cells within the ULA are centrifuged and due to the low attachment to the bottom surface, they can aggregate and form a 3D spheroid. Adapted from28.

Glioblastoma spheroids grow up to 7 days till reaching their final characteristics, which will be measured using AnaSP, a Matlab tool for the image analysis of spheroids25. Several other morphological parameters can be measured with this software, by simply analyzing brightfield images acquired with a standard optical microscope25:

- the equivalent diameter is calculated considering the Feret’s diameter, which is a measure of a spheroid’s size along a specific direction26;

- the “roundness” is a measure of how close the shape of the 2D spheroid image approaches a circle (low values of roundness are due to the lack of compactness or “spheroidization”)26;

- the “solidity” is an indicator of the roughness of the spheroidal surface, and it is useful to assess spheroid’s regularity (tumor spheroids are considered regular if solidity values are higher than 0.90)26;

- the “circularity” is used to calculate the sphericity index (SI), as shown in the 2.9 and 2.10 equations, which determines how much the spheroid is close to a spherical geometry shape. According to the sphericity index values, tumor aggregates are classified as spherical (SI ≥ 0.90) or non-spherical shape (SI < 0.90)26.

|  | Cir=4π*AreaPerimeter2 | (2.9) |
| --- | --- | --- |

|  | SI= Cir | (2.10) |
| --- | --- | --- |

The spheroids are also analyzed at day 7 through Live & Dead assay following the same protocols described in Section 2.5.1. At day 7, GB spheroids are also fixed and marked using Phalloidin and DAPI staining as described in Section 2.5.2.1.

After having cultured spheroids into ULA, it is necessary to seed them into the three central microwells which constitute the cancer area of the microfluidic platform. They are simply taken from ULA and released into wells with a pipette as they are distinguishable to the human eye. Once seeded, it is important to control at the optical microscope that they have been located in the right place. The BBB-on-a-chip is incubated for 24 hours and perfused with half medium for HUVECs and half medium for Astrocytes through the syringe pump. The day after the Live & Dead assay is performed using the protocol explained in Section 2.5.1 to verify the viability of spheroids into the micro-bioreactor.

# Chapter 3

# Results and discussion

In this Third Chapter all the results concerning the experimental protocols described in Chapter 2 are reported. The results about BBB-on-chip biological validation, permeability and transendothelial electrical resistance (TEER) tests will be described and discussed. Moreover, the Glioblastoma (GB) spheroids production protocol will be analyzed including their integration inside the microfluidic platform.

# 3.1 2D BBB-on-chip microfluidic device validation

Figure 3.1 Leakage test shows that plasma treatment worked properly and the colored tracers fill adequately all the compartments of the BBB-on-chip.

The result of the leakage test described in Section 2.2 and reported in Figure 3.1 will be discussed.

A syringe pump is used to flux the yellow dye with a flowrate of 5 µL/min through the inlet 2 of the upper channel. During its path, it diffuses through the porous membrane into the adjacent channel, mixing with the red dye and reaching the microwells. The tracer takes about 10 min to diffuse into the bottom channel.

This test is important for different reasons:

- to verify that the liquid is able to cross the membrane and flow through the microchannels, reaching the wells, where 3D spheroids of tumor will be seeded;

- to be sure that the culture medium can reach the cells, allowing them to grow and reproduce within the microfluidic device;

- to test the irreversible seal between the two layers making sure there is no leakage of liquid.

The leakage test shows that the microfluidic device has been well sealed by the plasma treatment, so that the coloured tracers remain within the platform.

# 3.2 Biological Validation for BBB-on-chip results

As reported in Section 2.4, the obtainment of a HUVECs layer on top of the porous membrane and 2/3 Astrocytes with 1/3 Pericytes on the bottom, requires specific seeding protocols. Briefly, Astrocytes and Pericytes are seeded first, placed in the incubator, and allowed to properly attach to the membrane overnight. HUVECs are seeded on the opposite side of the membrane the next day. In the following paragraphs, results of the biological protocols described in Chapter 2 are presented and discussed.

## 3.2.1 Astrocytes and Pericytes seeding inside the BBB-on-chip

The following Figures 3.2 and 3.3 show the results of representative Live&Dead assays on pure cultures of Astrocytes and Pericytes seeded on the PETE membrane in the BBB-on-chip. Both cell lines properly attached and proliferated reaching confluence within 7 days from seeding. The staining with Calcein-AM, Hoechst and Propidium allows to visualize the cells and to assess their viability. The results are consistent for both cell lines and only a few cells are marked in red with Propidium and thus dead. This confirms that the BBB-on-chip designed and produced represents a suitable microenvironment for the culture and growth of the chosen cell lines reconstructing the brain side of the BBB.

Figure 3.2 Results of Live&Dead assay for Astrocytes after (a) 3 days and (b) 7 days from seeding. Cell nuclei marked in blue with Hoechst, live cells cytoplasm in green with Calcein-AM and cells in the final stage of apoptosis in red with Propidium.

Figure 3.3 Results of Live&Dead assay for Pericytes after (a) 3 days and (b) 7 days from seeding. Cell nuclei marked in blue with Hoechst, live cells cytoplasm in green with Calcein-AM and cells in the final stage of apoptosis in red with Propidium.

## 3.2.2 HUVECs seeding inside the BBB-on-chip

Figure 3.4 Vasculature compartment where a monolayer of HUVECs is seeded on the porous membrane. Cells cytoplasm of living HUVECs are marked in green with Calcein-AM at day 7 after seeding.

After a fibronectin coating, HUVECs are seeded into the top microchannel and allowed to attach to the porous membrane. Figure 3.4 shows a reconstruction of the entire cell culture chamber with cells marked with Calcein-AM. Results from a Live&Dead assay performed at day 7 are represented in Figure 3.5: cells are alive and confluent, resulting in a compact monolayer of endothelial cells. Medium flows from inlet 2 to outlet 2 providing nutrients and growth factors, miming the capillary flow.

Figure 3.5 Results of Live&Dead assay for HUVECs at day 7 seeded on the porous membrane of the top chamber. Cell nuclei marked in blue with Hoechst, live cell cytoplasms in green with Calcein-AM and cells in the final stage of apoptosis in red with Propidium.

## 3.2.3 Co-culture of Astrocytes and Pericytes inside the BBB-on-chip

Figure 3.6 Results of the cerebral cells biculture. Cell Trackers mark: (a) Astrocytes in red; (b) Pericytes in green; (c) Merge of the same area to distinguish both the lines seeded as 2/3 Astrocytes and 1/3 Pericytes.

When co-culturing Astrocytes and Pericytes (66 and 33% respectively) we pre-stained them using different live cell trackers in order to recognize each cell population. Astrocytes are marked in red using Deep Red CelltrackerTM and Pericytes in green using Green CellTrackerTM, as shown in Figure 3.6. After 24 hours from seeding, both cell lines are correctly adhering to the bottom side of the membrane and are distinguishable. The PETE membrane partially absorbs the fluorescent solution leading to a green background signal.

Cells were stained with colored cell trackers only for the validation experiments. All other tests were performed using normal cells. The medium in the microfluidic device is continuously perfused by the syringe pump connected to the inlet 1. The medium formulation is a mixture of 50-50% Pericytes and Astrocytes specific media, and the flow rate is 20 μL/h. Live&Dead assays at day 3 and 7 prove correct operations and maintenance of viability.

A reconstruction of the entire surface of the porous membrane is represented in Figure 3.7, showing that cells have filled the available space, reaching confluence in one week.

Figure 3.7 Cerebral compartment where a biculture of Astrocytes and Pericytes on the bottom chamber are seeded on the porous membrane. Cells cytoplasm of living cells are marked in green with Calcein-AM at day 7 after seeding.

## 3.2.4 Triculture model of the BBB-on-chip

Figure 3.8 Live&Dead assay on the whole BBB-on-a-Chip (a) Biculture of cerebral cells which constitutes the cerebral side; (b) Monoculture of endothelial cells which constitutes the capillary side.

The complete blood-brain-barrier in vitro model is a triculture composed of a monolayer of HUVECs on the top of the membrane and a layer of a co-culture of 2/3 Astrocytes and 1/3 Pericytes. In this final setup, the medium is supplied from the inlet 2 and diffuses through the membrane. Its optimized composition is 50% HUVECs and 50% Astrocyte medium. Figure 3.8 shows the Live&Dead assay results for the triculture model cultured for 7 days inside the device. The brain side is in row (a), showing that most cells are alive and only a few nuclei are marked in red with Propidium. Results for HUVECs are in row (b) and follow the same trend observed for Astrocytes and Pericytes.

Figure 3.9 BBB-on-chip triculture at day 7 the overlapping of the two compartments: (a) Restriction related to the inlet 2; (b) One of the three central microchannels.

A lower magnification image of the microdevice is shown in Figure 3.9, highlighting the proximity of the two compartments and their correct complete coverage of the membrane. HUVECs are easily distinguishable thanks to their characteristic spindle-like and elongated shape.

Figure 3.10 BBB-on-chip triculture at day 7: (a) Brain cells nuclei are marked in blue; (b) Brain cells are marked in green with CellTrackerTM; (c) HUVEC are marked in red; (d) Merge of the two previous images with the whole three-culture model.

As a further validation, we used HUVECs stably expressing a red fluorescent cytoplasmatic protein and cerebral cells marked in green with CellTracker™ as explained in Section 3.2.1. Representative images are shown in Figure 3.10.

These results prove that our BBB-on-chip model is fully validated from a biological point of view, as the viability tests lead to optimum results for this three-culture model.

Figure 3.11 Phalloidin & DAPI Staining on the BBB-on-Chip, where: (a) DAPI marks all nuclei in blue; (b) Phalloidin marks F-actin filaments in red; (c) Merge of the two previous images.

Additional Phalloidin and DAPI staining (reported in Figure 3.11) highlights F-actin filaments marked in red and cell nuclei in blue 7 days after seeding. All cell types maintain their typical morphology and cover the entire cell culture chamber over and under the PETE porous membrane.

Figure 3.12 IF staining of HUVECs against CD31 on the BBB-on-Chip model 7 days from seeding. (a) DAPI marks all nuclei in blue; (b) CD-31 monoclonal antibody marks endothelial barrier in green; (c) Merge of the two previous images.

CD31 is a commonly used marker for endothelial cells, including those in the BBB, and indicates the formation of tight junctions and adherence junctions between endothelial cells, essential for the selective permeability of the BBB. A representative IF analysis against CD31 on HUVECs is presented in Figure 3.12 and confirms the correct endothelial phenotype and the formation of tight junctions within the chip, mimicking the physiological characteristics of the BBB. Moreover, the presence of CD31 staining suggests the integrity of the endothelial layer, further validating the fidelity of the BBB model for drug permeability studies, disease modeling, and other applications requiring an accurate representation of the blood-brain barrier.

Astrocytes are key components of the neurovascular unit, playing crucial roles in regulating BBB permeability, maintaining homeostasis, and modulating neuroinflammatory responses. We thus evaluated the presence of GFAP (Glial Fibrillary Acidic Protein) in the brain side of the BBB model to confirm the presence of Astrocytes, their functionality, and activation status. GFAP is a well-established marker for Astrocytes and provides insights into their dynamic interactions between with endothelial cells within the BBB, reflecting the complex crosstalk essential for BBB integrity and function. Representative images from our IF analysis against GFAP on Astrocytes are presented in Figure 3.13.

Figure 3.13 IF staining on the BBB-on-Chip, where: (a) DAPI marks all nuclei in blue; (b) GFAP monoclonal antibody marks Astrocytes in magenta; (c) Phalloidin marks F-actin filaments in orange; (d) Merge of the three previous images.

Figure 3.14 reports images for a double staining against CD31 and GFAP in the BBB-on-chip. This further confirms the formation of the compact CD31+ endothelial layer on top of the PETE membrane and the presence of GFAP+ Astrocytes on the opposite side 7 days from seeding.

Figure 3.14 IF staining on the BBB-on-Chip, where: (a) DAPI marks all nuclei in blue; (b) CD31 monoclonal antibody marks HUVEC in green; (c) GFAP monoclonal antibody marks Astrocytes in magenta; (d) Merge of the three previous images.

# 3.3 Results for permeability test

Validating the integrity and functionality of our BBB in vitro model is essential for ensuring its reliability and relevance in mimicking the physiological properties of the in vivo BBB. Permeability measurements serve as a critical parameter for assessing the fidelity of the models and their suitability for research purposes. Considering our goals, it is fundamental to measure times and concentrations reached in the tumor area of the device.

Measures must be performed on a mature barrier, at least at day 7 from seeding under continuous flow of medium through the syringe pump. This latter is required for proper elongation of endothelial cells in the direction of the flow resembling native tissue in contact with the blood flux and the main responsible of the decreased permeability in vivo.

To perform the analysis, FITC-conjugated dextrans of different molecular weights (40, 70, 150 kDa) are perfused into the top chamber, from inlet 2 to outlet 2. The fluorescence is monitored through a microscope using the GFP filter, capturing pictures of the three wells sequentially and then measuring the intensity with ImageJ. During operation, the fluorescent dextrans correctly fill the three wells, accumulating and slowly increasing their fluorescence intensity. The software analyzes specific regions of interest (ROI) of the image (selected at the center of each microwell), and we extracted the mean intensity value. The same test is performed using a device without cells as experimental control and a BBB-on-chip with only the monolayer of HUVEC to evaluate potential changes in permeability. We correctly observe an increase in the time required for the dextrans to fill the microwells as a function of their molecular weight: higher molecular weights correlate with increased difficulty in crossing the 3 μm pore membrane. Moreover, the permeability decreases with the addition of more cell layers.

The permeability value is calculated using the formula reported in Section 2.7.1 and the results, averaged over the values obtained for every experiment, are shown in Table 3.1. As observed, permeability is higher in the device without cells, and decreases by an order of magnitude with just a monolayer of HUVECs.

Table 3.1 Mean experimental permeability values in [cm/s] with their standard deviation (SD).

|  | No cells cms | Mono-culture cms | Three-culture cms |
| --- | --- | --- | --- |
| 40 KDa | 3.08 ⋅ 10-4 ± 7.57 ⋅10-6 | 9.90 ⋅ 10-5 ± 1.53 ⋅10-5 | 7.06 ⋅ 10-5 ± 1.23 ⋅10-5 |
| 70 KDa | 2.57 ⋅ 10-4 ± 5.67 ⋅10-5 | 9.03 ⋅ 10-5 ± 1.58 ⋅10-5 | 5.90 ⋅ 10-5 ± 7.32 ⋅10-6 |
| 150 KDa | 2.12 ⋅ 10-4 ± 1.73 ⋅10-6 | 8.95 ⋅ 10-5 ± 5.53 ⋅10-6 | 5.00 ⋅ 10-5 ± 1.77 ⋅10-7 |

Figure 3.15 Permeability tests. Permeability values for the Control (empty device), Mono-culture with HUVECs and Three-culture model are reported for the 3 dextrans tested: (a) 40 kDa ; (b) 70; (c) 150 kDa. All values are mean ± SEM from at least 3 independent experiments. **p<0.01, ***p<0.001 and ****p < 0.0001 vs control.

Figure 3.15 plots the results for the three molecular weights and experimental conditions tested, showing the significant decreases in permeability between the control device (no cells), the device with HUVECs alone and the three-culture BBB-on-chip. This qualitative observation is supported by paired t-tests conducted using GraphPad, showing a statistically significant difference between the groups of data.

# 3.4 Results for TEER experiments

TEER is used to assess the integrity and barrier function in in vitro models of the BBB. The TEER of the BBB-on-chip are calculated as detailed in Section 2.7.2 considering a membrane area of 20.745 mm2, and reported in Table 3.2.

Table 3.2 TEER values calculated at different time point after cell seeding. The values are expressed in [Ohm ⋅ cm2].

|  | Day 1 | Day 3 | Day 7 |
| --- | --- | --- | --- |
| TEER value | 85 ± 21.21 | 182 ± 14.14 | 337 ± 35.36 |

At day 1 the TEER value agrees with literature data23 for an early-stage BBB model, as already mentioned in Chapter 2, and it increases during time, reaching a final value of 337 Ohm*cm2 after 7 days. The values of reference for TEER measurement vary depending on factors such as the species, cell type, and experimental conditions. Generally, TEER values for a healthy BBB range from a few hundreds to several thousands Ohms per square centimeter (Ω·cm2).

These values are summarized in Figure 3.16.

Figure 3.16 TEER values at day 1, 3 and 7 for the three-culture on the BBB-on-chip. All values are mean ± SEM from at least 3 independent experiments. *p<0.05 vs control.

p-value = 0.0131

p-value = 0.0328

# 3.5 3D GelMA BBB validation results

To better mimic the 3D environment of the neurovascular compartment we attempted to create a GelMA-based construct. Our preliminary experiments show promising results, enabling long term culture for up to 30 days, but also highlight some limitations. A representative Live&Dead assay for Astrocytes (AM) and Pericytes (PM) seeded in the GelMA hydrogel 7 and 14 days from seeding is shown in Figure 3.17. Although the viability is consistently high, at day 7 cells still have a spherical shape, indicative of a slow growth rate. Astrocytes begin to slightly stretch only at day 14, while Pericytes form clusters, indicating cell distress.

Figure 3.17 Results of Live&Dead assay for Astrocytes and Pericytes growing in GelMA. (a) Results at day 7 after seeding; (b) Results after 14 days from seeding.

After 30 days, as shown in Figure 3.18, despite the presence of some dead cells (which cannot be removed from the construct with the normal operation of cell culture media), there is a significant increase in the number of Astrocytes, indicating the formation of a three-dimensional construct within GelMA. On the contrary Pericytes at day 30 show the same arrangement of day 14: they are a few and form clusters.

Figure 3.18 Results of Live&Dead assay for Astrocytes growing in GelMA at day 30 from seeding.

Figure 3.19 shows representative images of a Phalloidin and DAPI staining at day 30. Red stained F-actin filaments confirm that Astrocytes assumed their typical elongated shape within the 3D GelMA matrix.

Although the 3D environment of GelMA lowers cell growth rates, it more realistically mimics the in vivo thickness of the blood-brain barrier. However, the longer maturation times required might be a limiting factor for medical/therapeutic purposes.

Figure 3.19 Astrocytes culture in GelMA construct at day 30 marked with phalloidin to visualize the F-actin filaments in red and DAPI to visualize the cell nuclei in blue at different magnification: (a) 20X and (b) 10X.

# 3.6 Glioblastoma spheroids results

In this paragraph, results of Glioblastoma (GB) spheroids characterization are described. Spheroids are first cultured within Ultra Low Adhesion plates (ULA) and then they have been seeded into the cancer area of the BBB-on-chip.

## 3.6.1 GB spheroid characterization

Growth and morphology of U87 spheroids are monitored over 7 days. The spheroid size, solidity, and sphericity index (SI) were calculated. Images are taken in brightfield, making sure the photo is well contrasted, and then AnaSP analyzes the typical characteristics.

Observing the graphs in Figure 3.20, it can be observed that the equivalent diameter increases from a value of 200 μm at day 2, up to a value of about 700 μm (reaching peaks of 800 μm) at day 7. The solidity also increases during days which means that the regularity of spheroid’s shape increases, while compactness decreases because of growth effects. Sphericity index is nearly 1, confirming a spherical shape.

Figure 3.20 Spheroids’ characterization through AnaSP: (a) Equivalent diameter; (b) Solidity; (c) Compactness; (d) Sfericity Index.

At day 7 a Live&Dead assay and a Phalloidin & DAPI staining are performed (Figure 3.21 and Figure 3.22), showing respectively the viability of all cells and their actin filament’s structures.

Figure 3.21 3D Glioblastoma spheroids cultured in ULA at day 7: results for Live&Dead assay at different magnification. Cell nuclei marked in blue with Hoechst, cell cytoplasm marked in green with Calcein-AM and cells in the final stage of apoptosis marked in red with Propidium.

Figure 3.22 3D Glioblastoma spheroids cultured in ULA at day 7 marked with phalloidin to visualize the F-actin filaments in red and DAPI to visualize the cell nuclei in blue at different magnification: 10X and 20X.

## 3.6.2 Spheroids experiment in the BBB-on-chip

At day 7 from formation, spheroids are carefully transferred from the ULA surface and seeded into the three central microwells, which constitute the cancer area of the BBB-on-chip, as shown in Figure 3.23. These preliminary experiments are useful for understanding whether the spheroids remain inside the microwell when subjected to the flux of medium and whether they maintain their viability over time. Medium is supplied through the syringe pump with a flow rate of 5 μL/min and the device is incubated overnight. After 24 hours, a Live&Dead assay is performed confirming that GB spheroids maintain their viability within the microwells of the BBB-on-chip, thus mimicking the in vivo tumor site. The ultimate objective is to complete this experiment by flowing a drug loaded with monocytes through the syringe pump. This drug must cross the BBB-on-chip and genetically influence the glioblastoma cells’ DNA, rendering them more receptive to standard chemotherapy treatment.

Figure 3.23 (a) Cancer area of the micro-bioreactor where spheroids are seeded. (b) Live&Dead viability test results on GB spheroids into the three microwells after 24 hours after the starting of the experiments.

# Conclusions

The thesis project involves the design and validation of a PDMS microfluidic platform that reproduces the main characteristics of the blood-brain-barrier while also integrating the tumor site. The goal of the work is to develop a single, small, efficient and inexpensive device to study new strategies to treat Glioblastoma (GB) which is the most aggressive and fast-growing brain tumor. After dismissing the preliminary idea of culture cells inside a GelMA hydrogel construct, the BBB-on-chip is realized using a PETE porous membrane (with 3 μm diameter pores), which separates the brain compartment from the vascular channel, where cerebral and endothelial cells are seeded, respectively.

The geometry of the BBB-on-chip is designed using AutoCAD®, the Master mold is fabricated through Photolithography process and then the microfluidic device is realized in PDMS via Replica molding. Since the two PDMS layers, which recreate the Blood and the Brain compartments, are sealed together through a plasma treatment, a leakage test is performed to test the hydraulic seal: the microfluidic device perfectly resists to a flow rate of 5 μL/min.

The seeding protocol of the triculture composed of HUVECs, Astrocytes and Pericytes has been validated. Following the biological experimental campaign, which includes Live&Dead assay and Immunofluorescence staining within the device, paracellular barrier characteristics such as permeability and Trans Endothelial Electrical Resistance (TEER) have been calculated. The permeability test results have yielded consistent and significant data, showing that permeability values decrease with increasing cell layers attached to the porous membrane and with increasing molecular weight of the dextran tracers used.  A porous membrane with 3 μm diameter pores is deemed sufficient for the specific anticancer therapy under analysis, but pore dimensions can be reduced to 0.4 μm, resulting in even lower permeability values. TEER measurements indicate that as the chip approaches confluency, the calculated TEER values increase. Both permeability and TEER values obtained in this work are optimum and comparable to those found in the literature. A protocol for obtaining GB spheroids using ULA plates has been developed and validated. The spheroids are characterized and analyzed through Live&Dead assay and analysis of imaging.

Once the BBB-on-a-chip has been validated, 3D Glioblastoma spheroids are seeded into the cancer area of the microfluidic platform, showing promising viability results. Spheroids can live and grow within the three central microwells of the same device. In conclusion, the BBB-on-a-chip integrated with the glioblastoma tumor site can be exploited to study innovative treatment strategies which involve loaded monocytes that would genetically change tumor cells’ DNA, making them more receptive to standard chemotherapy.

# References

1.	Squires TM, Quake SR. (2005). Microfluidics: Fluid Physics at the Nanoliter Scale.

2.	Beebe, D.J., Mensing, G.A. & Walker, G. M. (2002). Physics and Applications of Microfluidics in Biology. Annu. Rev. Biomed. Eng. 4: 261-86.

3.	Zhang JXJ, Hoshino K. (2014). Microfluidics and Micro Total Analytical Systems. In: Molecular Sensors and Nanodevices. Elsevier: 103-168.

4.	Shakeri A, Khan S, Didar TF (2021). Conventional and emerging strategies for the fabrication and functionalization of PDMS-based microfluidic devices. Lab Chip. 21(16): 3053-3075.

5.	Breslauer DN, Lee PJ, Lee LP (2006). Microfluidics-based systems biology. Mol Biosyst. 2(2): 97-112.

6.	Sia SK, Whitesides GM (2003). Microfluidic devices fabricated in poly(dimethylsiloxane) for biological studies. Electrophoresis. 24(21): 3563-3576.

7.	Sito web: global.samsungdisplay.com

8.	Weibel DB, DiLuzio WR, Whitesides GM. (2007). Microfabrication meets microbiology. Nat Rev Microbiol. 5(3): 209-218.

9.	Sito web: https://plasmatreatment.co.uk/

10.	Sito web: www.henniker-scientific.com

11.	Bhattacharya S, Datta A, Berg JM, Gangopadhyay S. (2005). Studies on surface wettability of poly(dimethyl) siloxane (PDMS) and glass under oxygen-plasma treatment and correlation with bond strength. Journal of Microelectromechanical Systems. 14(3): 590-597.

12.	Ruiz-López E, Schuhmacher AJ. (2021). Transportation of single-domain antibodies through the blood–brain barrier. Biomolecules. 11(8).

13.	Sivandzade F, Cucullo L. (2018). In-vitro blood–brain barrier modeling: A review of modern and fast-advancing technologies. Journal of Cerebral Blood Flow and Metabolism. 38(10): 1667-1681.

14.	Imola Wilhelm, Csilla Fazakas, and Istvan A. Krizbai. (2011). In vitro models of the blood-brain barrier.

15.	Bi W, Cai S, Lei T, Wang L. (2023). Implementation of blood-brain barrier on microfluidic chip: Recent advance and future prospects. Ageing Res Rev. 87.

16.	Chen X, Liu C, Muok L, Zeng C, Li Y. (2021). Dynamic 3d on-chip bbb model design, development, and applications in neurological diseases. Cells. 10(11).

17.	Wirsching HG, Weller M. (2016). Glioblastoma. In: Malignant Brain Tumors: State-of-the-Art Treatment. Springer International Publishing. 265-288.

18.	Da Ros M, De Gregorio V, Iorio AL, et al. (2018). Glioblastoma chemoresistance: the double play by microenvironment and blood-brain barrier. Int J Mol Sci. 19(10).

19.	Nunes AS, Barros AS, Costa EC, Moreira AF, Correia IJ. (2019). 3D tumor spheroids as in vitro models to mimic in vivo human solid tumors resistance to therapeutic drugs. Biotechnol Bioeng. 116(1): 206-226.

20.	Costa EC, de Melo-Diogo D, Moreira AF, Carvalho MP, Correia IJ. (2018). Spheroids Formation on Non-Adhesive Surfaces by Liquid Overlay Technique: Considerations and Practical Approaches. Biotechnol J. 13(1).

21.	Białkowska K, Komorowski P, Bryszewska M, Miłowska K. (2020). Spheroids as a type of three-dimensional cell cultures - examples of methods of preparation and the most important application. Int J Mol Sci. 21(17): 1-17.

22.	Bova L, Maggiotto F, Micheli S, et al. (2023). A Porous Gelatin Methacrylate-Based Material for 3D Cell-Laden Constructs. Macromol Biosci. 23(2).

23.	Wang JD, Khafagy ES, Khanafer K, Takayama S, Elsayed MEH. (2016). Organization of Endothelial Cells, Pericytes, and Astrocytes into a 3D Microfluidic in Vitro Model of the Blood-Brain Barrier. Mol Pharm.13(3): 895-906.

24.	Gaillard PJ, Gerrit De Boer A. (2000). Relationship between Permeability Status of the Blood-Brain Barrier and in Vitro Permeability Coefficient of a Drug. 12. www.elsevier.nl/locate/ejps

25.	Piccinini F. (2015). AnaSP: A software suite for automatic image analysis of multicellular spheroids. Comput Methods Programs Biomed. 119(1): 43-52.

26.	Amaral RLF, Miranda M, Marcato PD, Swiech K. (2017). Comparative analysis of 3D bladder tumor spheroids obtained by forced floating and hanging drop methods for drug screening. Front Physiol. 8.

27.	Sito web: harrickplasma.com

28.	Sito web: sigmaaldrich.com